# RegJP2000 Version 1.0 (DRAFT VERSION)

Bob Courtney

Geological Survey of Canada

Natural Resources Canada

Bob.courtney@nrcan.gc.ca

January 20, 2010

## Table of Contents

RegJP2000 Version 1.0 (DRAFT VERSION)	1

Table of Contents	2

Introduction	3

Intellectual and Property Rights	6

Installation	7

Removal	7

Operation	8

Startup	8

Convert SEGY to JP2 :	8

Main Tab: Adding files to the Input Process List	8

Spectra Tab Page : Exploring Input SEGY File	12

Convert SEGY to SGYJP2 Tab Page:	17

Convert JP2 to SEGY :	25

Appendix I  - Storage and XML Schema	26

SEGY Header Schema	26

Horizon Schema	33

Routines to Manipulate JPEG2000 Box Data	36

## Introduction

RegJP2000 is a Windows XP program that is used to register scanned seismic and sidescan sections stored in JPEG2000 format and  project them into SEGY format for subsequent processing. The use of JPEG2000 makes the handling of very large images very quick and easy.

Image sizes in excess of 6000 x 300,000  pixels can be handled on most desktop PC’s (100 ft rolls of scanned analog section).

These SEGY files are often subsequently converted into SEGY-encoded JPEG 2000 format. These SEGY-encoded JPEG 2000 files can viewed simply as a grayscale image in most commonly available JPEG 2000 viewers (e.g., Adobe Photoshop, LizardTech,  Corel draw, etc.) .  However, these files contain much more; they contain all the trace information derived from the SEGY header data encoded as XML,and stored directly in the file. This information can be recovered and reconverted back into the original SEGY. An enhanced viewer that uses these extended capabilities is available through the GSC.

RegJP2000 represents the major component of a process ( scan ->convert/QC->register)  to convert analog sections into internet-ready, geographically-registered data objects as shown in the lower part of the following diagram.

The attempt here is to implement  a database- and internet-ready  framework that will allow researchers and others to easily exchange and archive both sonar data and their interpretations.   Our experience has suggested, that at least for single channel high-resolution seismic and sidescan data, scientists much prefer working with these JPEG2000-based digital sections rather than with the original paper sections.

JPEG2000 encoding of seismic and sidescan offers the capability to encode SEGY data efficiently (typical compression of 10:1 though to 40:1), offer quick evaluation of data quality through off-the-shelf image viewing technology and, importantly, afford a framework for embedding value-added interpretations to accompany the data. These files may contain 100,000’s of traces efficiently without undue allocation of system resources.

The adoption of a standard, open structure framework for storage allows this work to be leveraged by other technologies developed for dissemination of imagery stored in JPEG 2000, such as image servers and similar types of technologies. For example, the development of the JPIP internet transport mechanism for efficient transfer of JPEG 2000 data over serial communication lines holds great promise for interactive viewing and manipulation of these large data objects over standard TCP/IP connections.

One of the notable advantages of this approach is the multi-resolution or multi-scale nature of the JPEG 2000 storage framework. In storing the data in a multi-resolution form,  near-optimal images of these data can be displayed as one zooms in and out with very little effort and very little load on computer resources. This feature makes the onscreen representation of the seismic data  very similar to what one would see viewing a printed seismic section, with the added advantage of course that these data and the related interpretations are geographically registered.

JPEG2000 uses an efficient entropy –based MQ bitstream encoder for data storage. Therefore white space compresses very efficiently (as it has no entropy). This feature allows us to store seismic data zero-padded for trace delay changes, yielding seamless imagery which would be prohibitive in size in an uncompressed form. On the conversion of JPEG2000 files back to SEGY, these zero-padded section are removed,  recovering the original trace delayed form.

## Intellectual and Property Rights

This program is the intellectual property of the Government of Canada. All rights to modify and distribute this software are retained by the Government of Canada.

"Intellectual Property" means all rights in respect of Confidential Information, any know-how, inventions and improvements related to Confidential Information, including without limitation, patents, copyrights, trade secrets, trade-marks, registered industrial designs, any applications for same and all rights therein.

"Confidential Information" means any information of a scientific or technical nature disclosed to, generated or acquired by the user in the course of carrying out the activities defined in this Agreement, whether oral or recorded in any form or medium and whether or not protectable by copyright.

We allow this program to be redistributed freely in its original form. However, please send me an email if you have a copy – I’ll ensure you get on a mailing list for upgrades.

Contact the author, Bob Courtney, Natural Resources Canada,  1 Challenger Drive , Dartmouth Nova Scotia B2Y 4A2 , to obtain the most recent version.

Unfortunately we have limited capacity to support needs of outside users. We do however encourage joint research agreements to extend this framework.

The RegJP2000 distribution package may be freely distributed as it contains no proprietary code. It does utilize Kakadu (www.kakadusoftware.com) , GSL (http://www.network-theory.com/gsl/)   and TChart ( www.steema.com) libraries which are distributed in compliance with each of their respective licensing agreements.

The product will undoubtedly evolve over time so it is prudent to contact the author to obtain the most recent version. Since this product is free, sometimes one must wait until time is available to address bugs, etc.

## Installation

We recommend that you download three separate programs if you wish to convert TIFF or PNM scanned files into JPEG2000 images for registration;

SectionJP2 - This tool is used to convert large PPM or TIFF files into JPEG2000 format. Hopefull it is simple enough to operarate without help.

Download at : ftp://ftp.nrcan.gc.ca/gsc/courtney/SectionJp2/SectionJP2.zip

QCJP2  - This tool is used to quality control JPEG2000 scanned sections. In addition, this application may be used to rotate, flip sections, tone balance sections, and export to BMP format at reduced scales

Download at :

ftp://ftp.nrcan.gc.ca/gsc/courtney/QCJP2/qcjp2.zip

RegJP2000 :  This program can be used to register and project scanned JPEG2000 images into SEGY format.

Download Program  at : ftp://ftp.nrcan.gc.ca/gsc/courtney/RegJP2000/RegJP2000.zip

Download a Sample Scanned Image in JPEG2000 format :

ftp://ftp.nrcan.gc.ca/gsc/courtney/RegJP2000/83030_AG_100FT_290_0148.jp2

## Removal

To remove the package, choose Control Panel from the Windows Start Menu. Double click on  Add or Remove Programs. The package name is RegJP2000_deployment.

Note : It is necessary to manually remove an existing package before a newer version can be installed.

## Operation

## This help file covers only the operation of the RegJP2000 program. Eventually help files will be written for SectopnJP2 and QCJP2,  but the use of these programs is very simple.

### Startup

Download a sample scanned JPEG2000 file from ftp://ftp.nrcan.gc.ca/gsc/courtney/RegJP2000/83030_AG_100FT_290_0148.jp2

Make sure that the scanned image  file is writeable. If it is not then the program cannot store registration data within the image.

Run RegJP20000 by double-clicking on the desktop icon or by choosing the program from the Start=> All Programs=>NRCan=>RegJP2000  menu selection.

Choose the desired file using the File=>Open menu seletion in the Main Window. Three widows, the Main, the Zoom and the Expedition windows,  will appear ( Fig 2)

Fig2 – Opening a JPEG2000 scan for the first time

Enter the expedition id, or choose from a pull-down list. Also enter the year of the expedition. These expedition ids are free-form strings and should be fairly open in format. At the GSC, we try to restrict the expedition ids to the standard nomenclature in our ED database.

By default, it is assume that the horizontal axis of the scanned section is referenced by GMT/Universal  time. In some instances, the horizontal axis will be trace number or fix number. Choose the appropriate format with the radio buttons.

Note :  We have extensively  tested only  time-referenced sections. If you are registering trace number-referenced sections and you encounter problems, get in contact with me.

Fig 3 – Input expedition code and year

When you have finished, click “Hide” to de-clutter the work space. This window can be viewed and edited at any time by choosing the Register=>View Cruise Parameters menu option in the main window.

Fig 4 – Main window

The main window initially shows the whole scanned image. Axes dimensions are given in pixels, so in the image above the section length is roughly   35,000/300 = 116  inches.  Typically at the GSC we scan our sections at 300 dpi on a continuous feed Xerox scanner which scan  section lengths in excess of 30 m (100 ft) in a few minutes .

It should be noted that this image is usually under-sampled and, therefore may not be very appealing.  In Fig 4, over 35,000 pixels of the original image  in the x-axis have been squeezed into  around 1000 pixels for display. You can clearly observe that the section did not feed completely straight into the scanner as the section tends to drift off towards the  bottom of the frame on  the right-hand side.

One can zoom into the image by clicking the left mouse button on the upper left-hand corner of the desired area on the display area of the main windw and drag out a “rubberband” box towards the lower right-hand corner of the box.

Zoom out  to the original bounds by clicking with left mouse button on the lower right hand corner and pulling a box towards the upper left hand corner.

The view can be panned by depressing the right mouse button on the image and dragging this position to change  the new desired view limits.

The aspect ratio of the section may be changed by grabbing the corners of the main window frame with the mouse and resizing the window with the fet mouse button depressed.

The red rectangle show the area displayed in the zoom box.

Fig. 5 – The zoom box

To start the program, double click on the desktop SEGYJp2  icon or the icon within  the Program Files/NRCan menu entry.

If all goes well, the  main window will appear of the desktop ( Fig 1).

Figure 1 - Main Window of  SEGYJp2

## Convert SEGY to JP2 :

### Main Tab: Adding files to the Input Process List

From the menu bar at the top of the page,  choose File => Open SEGY Files to add SEGY files to the process list. By default, the file selection box will look for files with a “.sgy“ extension , but that feature can be overridden to include all files in the file selector box.

The program will conduct some rudimentary tests on the chosen files to ensure that they are SEGY files. Most of the time these tests detect SEGY compliance-  in the odd instance a noncompliant file can pass through these tests and may well cause failure of the program.

It is assumed that the source SEGY file contains only one channel of information. That being said, the program will convert SEGY files containing multiple channels with the provision that the trace length and sampling times are the same for every channel. Since this program does not change any values in the SEGY trace headers that are stored in the jp2 file, images of these multichannel files may look somewhat confusing when viewed, a conversion of this multichannel form back into SEGY will recover the original state of the source SEGY file.

The file list may be cleared, or selection within list removed, by using the buttons under the list.

Scanning files

Summary data of the SEGY files added to the input process list can be generated by double-clicking on the file name in the list. :

Care must be taken if variables have been set in the Spectra or Convert SEGY to SGYJP2 tabbed pages (e.g., band pass filtering, envelope detection). These options are executed when the file is read such that the summary data will reflect these chosen processes.

If the show histogram of amplitudes checkbox is enabled, a plot of the CDF of the data against normalized maximum amplitude is generated. This plot is useful for determining outlier properties.

This plot can be panned by holding the right mouse button down and dragging the image. This plot can be zoomed by holding the left mouse button down and dragging a rectangle from its upper left corner to a lower right corner. The view may be reset by dragging a rectangle in the reverse direction, from the lower right corner to the upper left corner.

### Spectra Tab Page : Exploring The Input SEGY File

The Spectra tabbed page allows the user to visualize the trace data contained within the SEGY file and also to assess the effect of processing parameters. By default, the data in the selected file in the input process list is displayed, or the first file  in the list if no file have been chosen.

The toolbar located above the graph is a standard plug-in from the graphics module, TChart (www.steema.com). This allows the most features of the plot to be customized, data within the plot saved as text, XML, HTML, or Excel format, and the plot to be saved in a variety of graphic formats, copied to clipboard, printed, etc. . It can even used to rotate the plot in3-D ( why one would do this, I don't know! ). Please play with the settings.

Get Next SEGY File : choose the next file in the input list.

See Next Trace: move to the next trace in the current SEGY file

Compute Single Spectra :

This button calculates and displays a single FFT spectra of the current trace. No averaging of successive trace spectra is performed, nor is any spectral averaging within the single trace estimate performed. And we all know what this means about the expected variance in the spectral estimates.

Bandpass Filter Settings :

Hi cut and low-cut filter settings are next applied along with a taper length, which specifies the frequency interval over which the filter amplitude varies from 0 to 1. The filter is implemented as a zero phase, mixed radix FFT-based filter. Future plans include different filter tapers for the low and top ends.

If the hi cut value is greater than the Nyquist frequency for the times series, then the hi cut value defaults to the Nyquist.

Compute filter spectral response:

The spectral response of the band pass filter may be displayed by pressing this button.

Show Filtered and Unfiltered Traces:

This button is used to plot the input SEGY trace (red)  along with the trace filtered by the specified bandpass (green). This allows the user to graphically see and appreciate what the bandpass filter is  doing to the data.

The user can zoom in two phases of interest by rubber-banding a rectangle on the plot from top left to bottom right, holding the left mouse button down . The plot extents may be reset by rubber-banding in the opposite direction

Please refrain from using the magnifying glass in the toolbar, this option is used to change 3-D viewing perspective. If by chance you happen to put the plot in 3-D mode, toggle the 3-D button on the toolbar to return to 2-D mode.

Envelope detection:

Often with broad band sources ( e.g., high-resolution boomer data), echo sounder type sources ( e.g. . Knudsen chirp) or high noise to signal situations, the display of data is improved by calculating the envelope of the trace.

Envelope detection in this program is executed through a convolution-based approach.

After we calculate envelope of the trace data, we apply a transform of the form

y=xα

where x is the envelope detected amplitude, y is the transformed variable, and α is the envelope exponent.

We apply this transform for two reasons:

- The transform applies some amplitude compression such that grayscale visualizations of the data show detail in both high and low amplitude parts of the section.

- More importantly, if the original trace contains  random environmental or other kind of noise with the normal distribution, then the envelope of this noise would have a Rayleigh statistic.

A Rayleigh statistic is what statisticians call a heavy-tailed distribution; that is, there are relatively numerous instances of high amplitudes; these effects manifest as speckle in images ( a common characteristic of SAR type imagery).

In the conversion of trace data to wavelet-based JPEG 2000, we prefer a data distribution with a Gaussian statistic as optimal compression and image fidelity is achieved with normally distributed data.

Numerical modeling has shown that a choice of α = 0.6 will transform a Rayleigh distribution to a distribution very similar to that of a non-zero mean Gaussian. Matlab code demonstrating this principle is available from the author.

Envelope Exponent :

Choose a value from 0.4 to 1.0. We recommend leaving the value at the default of 0.6 for the reasons outlined above.

Show filtered trace envelope:

Click this button to show the transformed envelope of the band-passed waveform data. If the envelope exponent is changed, the display will interactively reflect these changes.

Clipping:

In an ideal world, sonar data is uncorrupted by sound coming from other sources. In practice, however especially in the high-resolution game where multiple sources are often the water, seismic data are overprinted by unwanted  high amplitude noise, these noise sources can be both incoherent and coherent in nature.

In JPEG 2000 compression, data can be expressed with variable bit depths ranging from 1 to 32 bits for this implementation. We desire that the prescribed bit depth is used to span the waveform amplitudes of interest; we do not want to precisely define high amplitude spikes or other such data.

We implement histogram-based clipping as a means of removing high amplitude outliers that lay outside the normal variance of our data.

Clip above:

The clip above variable determines the percent of amplitudes in the data set that are passed unaltered into the JPEG 2000 compression engine. Amplitudes above this prescribed value are clipped and set to the maximum value determined by the normalized cumulative distribution function of the data.

In the spectra page, clipping is performed on a trace by trace basis but when we process the whole file, clipping is performed on the global aggregate distribution of data in the input file.

If this clip above value is set to 99%, for example, all data with amplitudes less than the 99% probability of the CDF pass unaltered, but all data values above the 99% CDF level are set to the clip level.

In practice, for envelope data, values of the clip above variable range between 99 and 100%, a starting value of 99.5% is reasonable.

The best way to check this value is to process the file and look at the resultant image. Images of the data, where the clipping level is too high, will appear too washed-out or faint, images of the data or the clipping level is set too low will appear oversaturated.

Show clipped filtered trace

This button will show the effect of clipping on the currently loaded trace. Changing the value of the clip threshold will interactively display the results on the screen.

### Convert SEGY to SGYJP2 Tab Page:

In this stage, the main work of converting all the files in the input list to sgyjp2  is done as a batch process. A typical conversion of a single SEGY file takes about five to 15 minutes so hopefully we can press the start button and come back in a few hours and everything will have been converted.

Expedition Metadata

Expedition-specific metadata is entered in the left-hand panel of this page. These data are saved as XML string type variables within the processed JPEG2000  file, and may be later on extracted and used to populate databases to describe compressed file holdings.

At the GSC, we use enumerated variables to describe the expedition ID, the data type, and the instrument type. These enumerated values can be chosen from the drop-down lists associated with each entry box. The expedition ID entry box is not kept up to date so users need to carefully enter expedition ID codes that are not found in the drop-down list.

Although all these enumerated values can be overridden in free-form text entry by the user, they should correspond at best as possible to enumerated or fixed values within our databases. This is a real concern for GSC or NRCAN employees.  Proper entry of the expedition ID code for example, is mandatory for subsequent merging of navigation data into the JPEG 2000 files, where the navigation is sourced from our databases.

Compression parameters:

The compression parameters are specified in the upper right hand corner of the page.

Compression parameters: maximum bits per sample

JPEG 2000 is not limited to fixed word lengths when storing data. It stores bit planes of the wavelet transform of the data and the number of bit planes can be tailored to best suit the desired resolution in the input data stream.

This numeric entry box specifies the maximum number of bits to been encoded for each trace sample within the input SEGY file. By default, the maximum bits per sample is 21 bits spawning a 126 dB of signal-to-noise - far exceeding any realistic dynamic range of data collected in the field.

SEGY data stored in floating point format have nominally 32 bits of dynamic range. But this is rarely, if ever, utilized within these files. We convert floating point data to integer form to encode these files in JPEG2000.  In this case, a floating point number for the maximum amplitude of the input file is found and stored in the JPEG 2000 file, and signal amplitudes are stored in scaled integer format at the prescribed bit rate.

Since this bit range will be applied relative to the maximum signal amplitude in the input file after bandpass filtering and clipping. So that this bit range corresponds to true dynamic range within the significant data.

Storing only 21 of 32 bits of a SEGY file immediately reduces the file size by a factor of 21/32, a 66% compression of the original file without loss above -126 db. If these JPEG 2000 files are subsequently converted back into SEGY format, they will be accurate within the bit depth used in the compression if a lossless compression ratio of 100% had been applied during the compression.

It should be noted that many commercial image viewing packages, such as those offered by Lizardtech and Adobe, may only be able to image data stored in eight bit and 16-bit formats. Choosing the maximum bits per sample equal to eight or 16 will allow the encoded JPEG 2000 files to be viewed as images in these commercial packages. Since 16 bits corresponds to 96 dB of signal to noise and that the 96 dB is applied from the maximum amplitude of the data set, 16 bits affords more resolution than is needed in most cases.

Compression parameters: calculated bits per sample

The input file is scanned and estimate is made of the bit-depth recorded within the raw data of the source file. If the calculated bit depth in the source file is less than the maximum bits per sample, then the data is stored at the lower bit rate. For example, during the 1990s, our onboard digitization systems recorded 12 bit data and typically this is the bit depth used to store the data.

However if the data is filtered or envelope-detected ( these are floating point operations), the will be converted to floating point data.

Compression parameters: prescribe compression ratio

The compression ratio is the size of the output JPEG 2000 file divided by the size of the input SEGY file expressed as a percentage. By default, a 10% compression is applied so that the output JPEG 2000 file will be roughly 10% of the size of the input file.

We have found very few artifacts in any data compressed at the 10% level, and we feel that this value is conservative for most seismic data sets. Even the value of 2 to 4% may prove sufficient as it usually  produces very little distortion of the seismic wavelets. The most distortion would be noticed at artificial boundaries within the data set, say if the traces were blanked down to a level just above the seafloor.

We would encourage the user to experiment with this setting and find the most appropriate setting for their application

Note: trace delays in SEGY files are used to generate a zero padded areas of the section so that features that trend across traces having different delays lineup in the JPEG 2000 image file. This zero padding effectively increases the raw size of the file and, at present, the compression ratio is applied to this expanded, padded file size. In these instances where the trace-delay-padded data file is much larger than the unpadded SEGY file, one may have to choose smaller values of the compression ratio to achieve the desired net compression. Hopefully this shortcoming will be addressed in future versions.

Compression parameters: prefiltering

The bandpass filtering strategy described in the previous section is applied to files on the stage during reading the input SEGY files. The values applied in the spectra page should have been transcribed into the numeric entry box is shown on this page. But to be safe checked to make sure these values in the high-cut, low-cut, and taper input boxes are appropriate. The bandpass filter is applied if the check box labeled prefilter trace is enabled.

Compression parameters: envelope exponent

The envelope exponent is applied to all band passed and clipped data.

I would try running the program using a value of 0.6 for reasons mentioned in the previous section on spectra. Although this rationale was based primarily on statistical characteristics of the envelope detected data, this exponent has proved useful simply as a signal conditioner that helps display dynamic range within the data set.

Compression parameters: clipping factor

The clipping factor is applied to remove outliers within the data set as a whole. For envelope detected data. A value of 99.5% or even higher seems to give good images of envelope-detected seismic data in JPEG 2000 viewers. I find lower values of between 95 to 99% are more appropriate for bipolar and half wave rectified versions.

So  try running the program using one of these values. And if the resulting image on inspection is too dark, then increase the clipping factor. If the resulting image is too faint in areas of interest, decrease the clipping factor. You'll soon get an idea of how the clipping factor affects image fidelity.

In the SEGYJp2_Viewer program, one can change the palette of the image, but we attempt here to choose an appropriate clipping factor such that, on first view,  the data looks well balanced in a tonal sense. Often the first view is what will be seen by those  scanning your inventory through network-based image viewers ( e.g., Lizardtech).

Compression parameters: envelope detection

Check off this box if you want the envelope-detected version of the input SEGY file. We find that the envelope detected waveform is highly appropriate for high-bandwidth boomer type data and also 3.5 kHz chirp type sounder. The resultant file from this choice has an “_envelope” suffix added to the root name of the input file name.

Important to note here is that whenever we take a bipolar waveform and transform it into a unipolar form, whether is be an envelope- detected or half wave rectified signal, the fundamental spectral content of the information changes.

Unipolar data has non-zero mean values whereas bipolar data tends to average out to zero. JPEG 2000 encoding of the data can be viewed as a series of nested averaging and differencing operations. As one zooms out of the JPEG 2000-coded image one looks at the averages of the data. When  one zooms in,  zero-mean differences are added back into the average yielding more detail.

Therefore, if one zooms out on a unipolar-encoded seismic image, they  would observe an averaged  picture with progressively less detail – much as would be observed as an observer backs away from a printed section.

However, if one zooms out on a bipolar image, the result in image would reveal progressively less and less information as the phases within the bipolar trace average out to zero.

Therefore for most viewing and interpretation purposes, one would likely choose either the envelope-detected option or the halfwave-rectified option.  For most data archive and transmission of the waveform data, one might choose the bipolar option.

One might produce bit-reduced version of the envelope and halfwave, at 8 bits for example, and distribute these  small but detailed versions to work with and save the full bit-depth bipolar version for the  archival purposes.

Compression parameters: half wave rectified

Check off this box if you desire to create a half wave rectified version of input SEGY file. The resultant file will have an “_halfwave” suffix added to the root name of the input SEGY file.

This half wave rectified version will produce image very similar to the types of data recorded on EPC graphic recorders in the past. The sharpness of the phase in the waveforms will be retained. And again, like the envelope detected waveform, the half wave rectified image has a nonzero mean, and therefore will display well as one zooms in and out.

Compression parameters: reversible transform

By default, the JPEG 2000 compression engine uses the 9 x 7 biorthogonal wavelet transforms, which are not precisely reversible. It is however, a relatively long filter and will provide better looking imagery  than once generated with the shorter 5x3 reversible transform pairs.

The 5 x 3 transform pairs will produce a perfectly reversible encoding so that data can exactly reproduced when converted back into SEGY from JPEG 2000, at the cost of the visual quality.

Generally this option should only be used if there are real concerns about reconstructing an exact replica of the input SEGY file. Since the signal to noise ratio SEGY data is generally quite low,  reconstruction errors are generally found below the noise threshold so that this concern is rarely one justified in real data sets.

Compression parameters: make a local copy for processing

At the GSC, many of our source files sit on network attached storage boxes on the local network, and sometimes the I/O performance of these boxes leave something to be desired. If the speed of compression as concern, this option may be enabled so that a copy of the file will be made locally to speed up the process.

## Convert JP2 to SEGY Tab Page:

Jp2-encoded SEGY files may be converted back into normal SEGY form. A list of files will be converted in btach mode back into SEGY form. If a subsection of a chosen file is desired, then double-click on the entry in the left-most list box and use the lower-right numeric entry box in order to choose new limits.

This conversion process cannot be applied to strictly image-type jp2 files that may been generated on a scanner for instance. These image files contain no encoded trace header information.

SEGY files that had been encoded with a band pass filter, an envelope detection or halfwave rectified will be reconstructed as floating point SEGY files regardless of their original format.

## Appendix I  - Storage and XML Schema

Trace data are stored in the image part of the JPEG2000 file in a compressed, variable bit length format.

SEGY file and trace header information and other related ancillary data (e.g., JPEG 2000 compression and bounding box data)  are stored in a user-defined box within the JPEG 2000 file as gzipped XML with the box label, “sxml”.

Interpretation data are stored in a separate box as gzipped XML with the box label, “hxml”.

These schema are subject to change. However, effort will be made to make newer versions backwards compatible with the versions presented here.

The schema were created in Visual Studio 2005 and consequently, the schema may be used to generate class libraries for I/O using the utility, XSD.exe for C++, C# or Visual Basic. I believe Java and other languages offer similar functionality.

A class library (written in C++ for Visual Studio 2005) for manipulating boxes within JPEG 2000 files is given in Appendix 2  and is available from the author.

We have limited capacity to provide assistance and support to outside users.

Copies of these schema are located in the Program Files\NRCan\SEGYJP2 directory.

### SEGY Header Schema

?xml version="1.0" encoding="utf-8"?>

<xs:schema xmlns:ns1="SEGY_rev_1" attributeFormDefault="unqualified" elementFormDefault="qualified" xmlns:xs="http://www.w3.org/2001/XMLSchema">

<xs:element name="JPEG2000_SEGY">

<xs:complexType>

<xs:sequence>

<xs:element name="FileInfo">

<xs:complexType>

<xs:all>

<xs:element name="file_name" type="xs:string" maxOccurs="1" minOccurs="1" />

<xs:element type="xs:string" name="instrument_type" maxOccurs="1" minOccurs="1" />

<xs:element name="instrument_comments" type="xs:string" minOccurs="0" maxOccurs="1" />

<xs:element name="jpeg2000_compression_parameters" type="JPEG2000CompressionParameters" maxOccurs="1" minOccurs="1">

</xs:element>

<xs:element name="bounding_box" type="BoundingBox" maxOccurs="1" minOccurs="1">

</xs:element>

<xs:element name="URI_x0020__to_digital_file" type="xs:anyURI" minOccurs="0" maxOccurs="1" />

<xs:element name="URI_to_scanned_image_file" type="xs:anyURI" minOccurs="0" maxOccurs="1" />

<xs:element name="input_byte_order" type="xs:boolean" minOccurs="1" maxOccurs="1" />

<xs:element name="expedition_id" type="xs:string" />

<xs:element name="NAP_metadata_profile" type="NAPMetadataProfile" minOccurs="0" maxOccurs="1" />

<xs:element name="input_file_contents" type="FileContents" minOccurs="1" maxOccurs="1" />

<xs:element name="expedition_comments" type="xs:string" maxOccurs="1" minOccurs="0" />

<xs:element name="scientist" type="xs:string" minOccurs="0" maxOccurs="1" />

<xs:element name="data_type" type="xs:string" />

<xs:element name="transducer" type="xs:string" />

</xs:all>

</xs:complexType>

</xs:element>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:complexType name="BoundingBox">

<xs:sequence>

<xs:element name="start_x0020_time" type="xs:dateTime" minOccurs="1" maxOccurs="1" />

<xs:element name="end_x0020_time" type="xs:dateTime" maxOccurs="1" minOccurs="1" />

<xs:element name="southern_latititude" type="xs:double" maxOccurs="1" minOccurs="1" />

<xs:element name="northern_latititude" type="xs:double" maxOccurs="1" minOccurs="1" />

<xs:element name="western_longitude" type="xs:double" maxOccurs="1" minOccurs="1" />

<xs:element name="eastern_longitude" type="xs:double" maxOccurs="1" minOccurs="1" />

<xs:element name="number_of_channels" type="xs:unsignedInt" maxOccurs="1" minOccurs="1" />

<xs:element name="number_of_traces" type="xs:unsignedInt" maxOccurs="1" minOccurs="1" />

<xs:element name="minimum_trace_time" type="xs:int" maxOccurs="1" minOccurs="1" />

<xs:element name="maximum_trace_time_x0020_" type="xs:int" maxOccurs="1" minOccurs="1" />

<xs:element name="number_of_samples_per_trace" type="xs:unsignedInt" />

<xs:any />

</xs:sequence>

</xs:complexType>

<xs:complexType name="BinaryFileHeader">

<xs:sequence>

<xs:element name="job_identification_number" type="xs:int" />

<xs:element name="line_number" type="xs:int">

</xs:element>

<xs:element name="reel_number" type="xs:int" />

<xs:element name="number_of_data_traces_per_ensemble" type="xs:short" />

<xs:element name="number_of_auxiliary_traces_per_ensemble" type="xs:short" />

<xs:element name="sample_interval_in_microseconds" type="xs:unsignedShort" />

<xs:element type="xs:unsignedShort" name="sample_interval_in_usec_of_original_field_recording" />

<xs:element name="number_of_samples_per_data_trace" type="xs:unsignedShort" />

<xs:element name="number_of_samples_per_data_trace_for_original_field_recording" type="xs:unsignedShort" />

<xs:element name="data_sample_format_code" type="xs:unsignedShort" />

<xs:element name="ensemble_fold" type="xs:unsignedShort" />

<xs:element name="trace_sorting_code" type="xs:short" />

<xs:element name="vertical_sum_code" type="xs:short" />

<xs:element name="sweep_frequency_at_start__x005F_x0028_Hz_x005F_x0029_" type="xs:unsignedShort" />

<xs:element name="sweep_frequency_at_end__x005F_x0028_Hz_x005F_x0029_" type="xs:unsignedShort" />

<xs:element name="sweep_length__x005F_x0028_ms_x005F_x0029_" type="xs:unsignedShort" />

<xs:element name="sweep_type_code" type="xs:unsignedShort" />

<xs:element name="trace_number_of_sweep_channel" type="xs:unsignedShort" />

<xs:element name="sweep_trace_taper_length_in_milliseconds_at_start" type="xs:unsignedShort" />

<xs:element name="sweep_trace_taper_length_in_milliseconds_at_end" type="xs:unsignedShort" />

<xs:element name="taper_type" type="xs:unsignedShort" />

<xs:element name="correlated_data_traces" type="xs:unsignedShort" />

<xs:element name="binary_gain_recovered" type="xs:unsignedShort" />

<xs:element name="amplitude_recovery_method" type="xs:unsignedShort" />

<xs:element name="measurement_system" type="xs:unsignedShort" />

<xs:element name="impulse_signal_polarity" type="xs:unsignedShort" />

<xs:element name="vibratory_polarity_code" type="xs:unsignedShort" />

<xs:element name="unassigned_3261_3500" minOccurs="0" maxOccurs="1">

<xs:simpleType>

<xs:restriction base="xs:base64Binary">

<xs:length value="240" />

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="SEG_Y_Format_Revision_Number" type="xs:unsignedShort" default="0" minOccurs="0" maxOccurs="1" />

<xs:element name="fixed_length_trace_flag" type="xs:unsignedShort" default="0" minOccurs="0" maxOccurs="1" />

<xs:element name="number_of_extended_Textual_files" type="xs:short" default="0" minOccurs="0" maxOccurs="1" />

<xs:element name="reservered">

<xs:simpleType>

<xs:restriction base="xs:base64Binary">

<xs:length value="94" />

</xs:restriction>

</xs:simpleType>

</xs:element>

</xs:sequence>

</xs:complexType>

<xs:complexType name="BinaryTrace">

<xs:sequence>

<xs:element name="trace_in_line" type="xs:unsignedInt" minOccurs="0" maxOccurs="1" />

<xs:element name="trace_in_file" type="xs:unsignedInt" minOccurs="0" maxOccurs="1" />

<xs:element name="orig_field_record" type="xs:unsignedInt" minOccurs="0" maxOccurs="1" />

<xs:element name="trace_in_orig_field_record" type="xs:unsignedInt" minOccurs="0" maxOccurs="1" />

<xs:element name="energy_source_point" type="xs:unsignedInt" minOccurs="0" maxOccurs="1" />

<xs:element name="ensemble" type="xs:int" minOccurs="0" maxOccurs="1" />

<xs:element name="trace_in_ensemble" type="xs:int" minOccurs="0" maxOccurs="1" />

<xs:element name="trace_id_code" type="xs:short" minOccurs="0" maxOccurs="1" />

<xs:element name="vertically_summed_traces" type="xs:short" minOccurs="0" maxOccurs="1" />

<xs:element name="horizontally_summed_traces" type="xs:short" minOccurs="0" maxOccurs="1" />

<xs:element name="data_use" type="xs:short" minOccurs="0" maxOccurs="1" />

<xs:element name="distance_from_tx_to_rx" type="xs:int" minOccurs="0" maxOccurs="1" />

<xs:element name="rx_elevation_" type="xs:int" minOccurs="0" maxOccurs="1" />

<xs:element name="surface_elev_at_tx" type="xs:int" minOccurs="0" maxOccurs="1" />

<xs:element name="tx_depth_below_surface" type="xs:int" minOccurs="0" maxOccurs="1" />

<xs:element name="datum_elev_at_rx" type="xs:int" minOccurs="0" maxOccurs="1" />

<xs:element name="datum_elev_at_tx" type="xs:int" minOccurs="0" maxOccurs="1" />

<xs:element name="water_depth_at_tx" type="xs:int" minOccurs="0" maxOccurs="1" />

<xs:element name="water_depth_at_rx" type="xs:int" minOccurs="0" maxOccurs="1" />

<xs:element name="scalar_multiplier_69-70" type="xs:short" minOccurs="0" maxOccurs="1" />

<xs:element name="scalar_multiplier_71-72" type="xs:short" minOccurs="0" maxOccurs="1" />

<xs:element name="tx_x" type="xs:int" minOccurs="0" maxOccurs="1" />

<xs:element name="tx_y" type="xs:int" minOccurs="0" maxOccurs="1" />

<xs:element name="rx_x" type="xs:int" minOccurs="0" maxOccurs="1" />

<xs:element name="rx_y" type="xs:int" minOccurs="0" maxOccurs="1" />

<xs:element name="coordinate_units" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="weathering_velocity" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="subweathering_velocity" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="uphole_time_at_tx" type="xs:unsignedShort" minOccurs="0" maxOccurs="1 " />

<xs:element name="uphole_time_at_rx" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="tx_static_correction_" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="rx_static_correction_" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="total_static_correction_" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="lag_time_A" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="lag_time_B" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="delay_record_time" type="xs:short" minOccurs="0" maxOccurs="1" />

<xs:element name="mute_start_time" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="mute_end_time" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="number_of_samples_in_trace" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="sample_interval__x005F_x0028_usec_x005F_x0029_" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="gain_type_of_field_instruments" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="instrument_gain_constant" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="instrument_initial_gain" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="correlated" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="sweep_freq_start" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="sweep_freq_end" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="sweep_length__x005F_x0028_msec_x005F_x0029_" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="sweep_type" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="sweep_taper_length_start" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="sweep_taper_length_end" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="taper_type" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="antialias_filter_frequency" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="antialias_filter_slope" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="notch_filter_freq" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="notch_filter_slope" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="low-cut_freq" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="high_cut_freq" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="low-cut_slope" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="high-cut_slope" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="year_data_recorded" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="day" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="hr" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="min" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="sec" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="msec" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="time_weighting_factor" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="group_number_of_roll_sw_1" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="group_number_of_tr1" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="group_number_of_last_trace" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="gap_size" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="over_travel" type="xs:unsignedShort" minOccurs="0" maxOccurs="1" />

<xs:element name="trace_data" minOccurs="0" maxOccurs="1">

<xs:simpleType>

<xs:restriction base="xs:base64Binary">

<xs:maxLength value="64000" />

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="taper" type="xs:short" minOccurs="0" maxOccurs="1" />

<xs:element name="TriggerInterval" type="xs:short" minOccurs="0" maxOccurs="1" />

<xs:element name="reserved">

<xs:simpleType>

<xs:restriction base="xs:base64Binary">

<xs:length value="58" />

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:any />

</xs:sequence>

<xs:attribute name="is_reference_header" type="xs:boolean" default="false" />

<xs:attribute name="trc_num" type="xs:int" />

</xs:complexType>

<xs:complexType name="TextualFileHeader">

<xs:sequence>

<xs:element name="text" minOccurs="40" maxOccurs="40">

<xs:complexType>

<xs:sequence>

<xs:element name="ascii_string">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:length value="80" />

</xs:restriction>

</xs:simpleType>

</xs:element>

</xs:sequence>

<xs:attribute name="line_number" type="xs:unsignedShort" />

</xs:complexType>

</xs:element>

<xs:element name="is_EBCDIC" type="xs:boolean" />

</xs:sequence>

<xs:attribute name="extended_header_number" type="xs:unsignedByte" />

</xs:complexType>

<xs:complexType name="JPEG2000CompressionParameters">

<xs:sequence>

<xs:element name="transform_offset" type="xs:double" />

<xs:element name="transform_gain" type="xs:double" />

<xs:element name="transform_function" type="xs:short" />

<xs:element name="number_of_significant_bits_in_input_stream" type="xs:unsignedByte" />

<xs:element type="xs:float" name="number_of_bits_per_sample_in_encoding" />

<xs:element name="rms_error_specified_in_encoding" type="xs:double" />

<xs:element name="input_trace_bipolar" type="xs:boolean" />

<xs:element name="significant_bits_at_top_of_word" type="xs:boolean" />

<xs:element name="minimum_trace_delay_x0020__x005F_x0028_msec_x005F_x0029_" type="xs:int" />

<xs:element name="padded_trace_length" type="xs:unsignedInt">

</xs:element>

<xs:element name="jp2000_wavelet_transform" type="xs:unsignedByte" />

</xs:sequence>

</xs:complexType>

<xs:complexType name="SEGYContents">

<xs:sequence>

<xs:element name="text_file_header" type="TextualFileHeader" minOccurs="1" />

<xs:element name="binary_file_header" type="BinaryFileHeader" minOccurs="1" maxOccurs="1" />

<xs:element name="binary_trace" type="BinaryTrace" minOccurs="1" maxOccurs="unbounded" />

<xs:element name="original_trace_converted_to_envelope" type="xs:boolean" minOccurs="0" maxOccurs="1" default="false" />

<xs:element name="envelope_exponent" type="xs:double" minOccurs="0" maxOccurs="1" default="1.0" />

<xs:element name="time_lines_embedded_in_trace_data" type="xs:boolean" default="false" maxOccurs="1" minOccurs="0" />

<xs:element name="horizontal_time_line_separation_msec" type="xs:double" minOccurs="0" maxOccurs="1" default="250" />

<xs:element name="vertical_time_line_separation_sec" type="xs:double" minOccurs="0" maxOccurs="1" default="600." />

<xs:element name="original_trace_converted_to_half_wave" type="xs:boolean" default="false" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

<xs:complexType name="NAPMetadataProfile">

<xs:sequence>

<xs:element name="to_x0020_be_x0020_defined" type="xs:string" />

</xs:sequence>

</xs:complexType>

<xs:complexType name="FileContents">

<xs:sequence>

<xs:element name="file_type" type="xs:string" />

<xs:element name="SEGY_contents" type="SEGYContents" minOccurs="0" maxOccurs="1" />

<xs:element name="xtf_contents" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence />

</xs:complexType>

</xs:element>

<xs:element name="gsf_contents" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence />

</xs:complexType>

</xs:element>

</xs:sequence>

</xs:complexType>

</xs:schema>

### Horizon Schema

Interpretations are stored in a separate box in structured XML. The intent here is to separate interpretations from the data, but keep them packaged together.

Although it has not yet been implemented, it is intended to offer the capacity to have multiple interpretations included with a given data set, offering the capacity to exchange and reinterpret data sets in the lab and over the Internet.

In the following schema, all X coordinates are sequential trace numbers and all Y coordinates are two-way travel times in milliseconds.

<?xml version="1.0" encoding="utf-8"?>

<xs:schema id="horizons" targetNamespace="http://tempuri.org/horizons.xsd" elementFormDefault="qualified" xmlns="http://tempuri.org/horizons.xsd" xmlns:mstns="http://tempuri.org/horizons.xsd" xmlns:xs="http://www.w3.org/2001/XMLSchema">

<xs:complexType name="horizon_x0020_pick">

<xs:sequence>

<xs:element name="x" type="xs:double" />

<xs:element name="y" type="xs:double" />

<xs:element name="line_break" type="xs:boolean" default="false" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

<xs:complexType name="horizon">

<xs:sequence>

<xs:element name="horizon_name" type="xs:string" minOccurs="1" maxOccurs="1" />

<xs:element name="description" type="xs:string" minOccurs="0" />

<xs:element name="color" type="xs:string" minOccurs="1" maxOccurs="1" />

<xs:element name="picks" type="horizon_x0020_pick" minOccurs="0" maxOccurs="unbounded" />

<xs:element name="date_x0020_created" type="xs:dateTime" minOccurs="0" />

<xs:element name="date_x0020_modified" type="xs:dateTime" />

<xs:element name="mapper" type="xs:string" minOccurs="0" />

</xs:sequence>

</xs:complexType>

<xs:complexType name="horizon_x0020_group">

<xs:sequence>

<xs:element name="source_x0020_sgyjp2_x0020_file" type="xs:string" />

<xs:element name="horizons" type="horizon" minOccurs="1" maxOccurs="unbounded" />

<xs:element name="group_x0020_name" type="xs:string" />

<xs:element name="point_x0020_markers" minOccurs="0" maxOccurs="unbounded">

<xs:complexType>

<xs:sequence>

<xs:element name="x_x0020_time" type="xs:double" />

<xs:element name="y_x0020_time" type="xs:double" />

<xs:element name="trace_x0020_number" type="xs:int" />

<xs:element name="comment" type="xs:string" />

<xs:element name="marker" minOccurs="0" maxOccurs="1">

<xs:complexType>

<xs:sequence>

<xs:element name="size" type="xs:double" />

<xs:element name="color" type="xs:int" />

<xs:element name="type" type="xs:string" />

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="lat" type="xs:double" minOccurs="0" maxOccurs="1" />

<xs:element name="lon" type="xs:double" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="frame_x0020_description" maxOccurs="unbounded" minOccurs="0">

<xs:complexType>

<xs:sequence>

<xs:element name="x_x0020_time_x0020_min" type="xs:double" />

<xs:element name="x_x0020_time_x0020_max" type="xs:double" />

<xs:element name="y_x0020_time_x0020_min" type="xs:double" />

<xs:element name="y_x0020_time_x0020_max" type="xs:double" />

<xs:element name="date_x0020_created" type="xs:dateTime" />

<xs:element name="orginal_x0020_author" type="xs:string" />

<xs:element name="last_x0020_date_x0020_modified" type="xs:dateTime" />

<xs:element name="last_x0020_author" type="xs:string" />

<xs:element name="description" type="xs:string" />

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="version" type="xs:string" minOccurs="0" maxOccurs="1" />

</xs:sequence>

</xs:complexType>

<xs:element name="horizon_x0020_top_x0020_level">

<xs:complexType>

<xs:sequence>

<xs:element name="group" type="horizon_x0020_group" minOccurs="1" maxOccurs="unbounded" />

</xs:sequence>

</xs:complexType>

</xs:element>

</xs:schema>

### Routines to Manipulate JPEG2000 Box Data

JPEG2000_Manage.h ( Visual Studio 2005 .NET)

#pragma once

using namespace System;

using namespace System::IO;

using namespace System::IO::Compression;

using namespace System::Runtime::InteropServices;

using namespace System::Xml;

using namespace System::Xml::Serialization;

using namespace System::Collections;

ref class  jp2_box_data

{

public:

unsigned int position;

unsigned int box_length;

String^ box_label;

Boolean xl_box;

Boolean zero_length_box;

};

#define IO_BUFFER_LENGTH 5000000; // define a 5 mbyte io buffer

ref class JPEG2000_Manage

{

public:

JPEG2000_Manage(void);

property String^ filename;

property unsigned int box_length;

property Boolean xl_box;

property Boolean zero_length_box;

property FileStream^ filestream;

property BinaryReader^ binaryreader;

property BinaryWriter^ binarywriter;

Boolean has_sxml_content(void); // check to see if it has SEGY formatted xml content

// open up a file for read write

Boolean open(void);  // file name preset with property filename;

Boolean open(String^);

Boolean open(Stream^ s);

Boolean close(void);

Boolean is_valid_jp2(void); // looks for "jP  " header box

// get the next box header - returns box label as String^ ; box length is in this->box_length; returns null string if failed or EOF

// leave the cuurent stream of file at start of box contents

String^ get_next_box_header(void);

String^ get_next_box_header(Stream^ s);

// skip over the box contents leaving the file postion at start of next jp2000 box

Boolean skip_box_contents(void);

Boolean skip_box_contents(Stream^ s);

// set filestream

void go_to_top_of_file(void);

// checks to see if at least one box with a given box_label exists

bool find_box(String^ box_label);

// fix zero length last box in file

bool fix_zero_length_box();

// write an serializable object to an xml file

static Boolean write_object_to_xml_text_file(Object^ input_object, String^ xml_text_file_name);

static Object^ read_object_from_xml_text_file(System::Type^ input_object_type, String^ xml_text_file_name);

// write a serializable object to a gzipped  xml file

static Boolean  write_object_to_xml_gzip_file(Object^ input_object, String^ xml_gzip_file_name);

static Object^  read_object_from_xml_gzip_file(System::Type^ output_object_type, String^ xml_gzip_file_name);

static System::Void read_object_from_xml_gzip_file(Object^ output_object, String^ xml_gzip_file_name);

// write an xml version of an object in gzipped format to a stream

Boolean write_object_to_xml_gzip_stream(Object^ input_object, Stream^ ms);

// write an xml version of an object in text format to a stream

Boolean write_object_to_xml_stream(Object^ input_object, Stream^ ms);

// write serializable object in gzipped xml format to a jp2000 compliant box at the current stream position

Boolean write_compressed_xml_box(Object^ input_object, String^ box_label, String^ gzip_xml_box_file_name);

// write serializable object in gzipped xml format to a jp2000 compliant box at the current stream position

Boolean write_compressed_xml_box(Object^ input_object, String^ box_label, Stream^ s);

// write serializable object  in readable xml format to a jp2000 compliant box in a file

Boolean write_xml_box(Object^ input_object, String^ box_label, String^ xml_box_file_name);

// write serializable object  in readable xml format to a jp2000 compliant box at the current stream position

Boolean write_xml_box(Object^ input_object, String^ box_label, Stream^ s);

// Read an xml object from a stream whether compressed or not

Object^ read_xml_box(System::Type^ input_object_type, String^ box_label , Stream^ s);

// Copy a jp2000 compliant  box from one stream to another

Boolean copy_box(Stream^ input_stream, Stream^ output_stream);

// get a summary list of jp2000 boxes in current file - creates an arraylist of type jp2_box_data

ArrayList^ get_box_data(void);

// compress current jp2 file

Boolean compress_existing_sxml_box(String^ new_file_name);

// does the file contain compressed xml content

Boolean has_compressed_sxml_content(void);

private:

Boolean input_file_is_open;

array<unsigned char,1>^ box_type;

array<unsigned char,1>^ box_XL;

void Write_Box_Header_to_MemoryStream(String ^box_label, BinaryWriter ^bw, MemoryStream ^ms);

};

JPEG2000_Manage.cpp ( Visual Studio 2005 .NET)

#include "StdAfx.h"

#include "stdio.h"

#include "JPEG2000_Manage.h"

#include "SEGY.h" // sxml header class

// utilities

System::UInt32 swap(UInt32 x)

{

// implement a swap

array<unsigned char,1>^ c =  BitConverter::GetBytes(x);

unsigned char tmp;

tmp = c[0];

c[0] = c[3];

c[3] = tmp;

tmp = c[1];

c[1] = c[2];

c[2] = tmp;

return BitConverter::ToUInt32(c,0);

}

System::UInt16 swap(UInt16 x)

{

// implement a swap

array<unsigned char,1>^ c =  BitConverter::GetBytes(x);

unsigned char tmp;

tmp = c[0];

c[0] = c[1];

c[1] = tmp;

return BitConverter::ToUInt16(c,0);

}

System::Int16 swap(Int16 x)

{

// implement a swap

array<unsigned char,1>^ c =  BitConverter::GetBytes(x);

unsigned char tmp;

tmp = c[0];

c[0] = c[1];

c[1] = tmp;

return BitConverter::ToInt16(c,0);

}

JPEG2000_Manage::JPEG2000_Manage(void)

{

this->filestream = nullptr;

this->binaryreader = nullptr;

this->filename = nullptr;

this->input_file_is_open = false;

this->box_length = -1;

}

System::Boolean JPEG2000_Manage::has_sxml_content(void)

{

return this->find_box("sxml");

}

System::Boolean JPEG2000_Manage::open(void)

{

if( this->filename == nullptr) return false;

this->input_file_is_open= false;

try

{

this->filestream = File::Open(this->filename,FileMode::OpenOrCreate);

}

catch (System::Exception^ )

{

return false;

}

try

{

this->binaryreader = gcnew BinaryReader(this->filestream);

}

catch ( System::Exception^)

{

return false;

}

try

{

this->binarywriter= gcnew BinaryWriter(this->filestream);

}

catch ( System::Exception^)

{

return false;

}

this->input_file_is_open = true;

return true;

}

System::Boolean JPEG2000_Manage::open(String^ input_name)

{

this->filename = String::Copy(input_name); // set filename

return this->open();

}

System::Boolean JPEG2000_Manage::open(Stream^ s)

{

this->filestream = (FileStream^) s;

try

{

this->binaryreader = gcnew BinaryReader(this->filestream);

}

catch ( System::Exception^)

{

return false;

}

try

{

this->binarywriter= gcnew BinaryWriter(this->filestream);

}

catch ( System::Exception^)

{

return false;

}

this->filename = "Stream input"; // set filename

return true;

}

String^ JPEG2000_Manage::get_next_box_header(void)

{

return this->get_next_box_header(this->filestream);

}

String^ JPEG2000_Manage::get_next_box_header(Stream ^s)

{

if ( s->Position >= s->Length ) return nullptr; // at eof

BinaryReader^ binaryreader = gcnew  BinaryReader(s);

// read the box header bytes

// jp2 are big-endian

box_length = swap(binaryreader->ReadUInt32()) ;

// set zero box length flag

this->zero_length_box = false;

if( !box_length)

{

this->zero_length_box = true;

this->box_length = s->Length - s->Position + 4;

}

// read box type

box_type = binaryreader->ReadBytes(4);

this->xl_box = false;

// read next eight bytes - note first 4 bytes should be all zeroes for 32 bit file sizes

if ( box_length == 1 )

{

this->xl_box = true;

box_XL = binaryreader->ReadBytes(8);

// byte swap part of word

for ( int i = 0; i < 4 ; i++ ) {

unsigned char tmp = box_XL[i];

if ( tmp != NULL )

{

// high bytes should be zero for 32 bit files lengths

return nullptr;

}

box_XL[i] = box_XL[7-i];

box_XL[7-i] = tmp;

}

// limit to 32 bit lengths; adjust length for preamble bytes

box_length =Convert::ToUInt32( BitConverter::ToUInt64(box_XL,0)) ;

}

// copy box_type into holding array - this is a real hack!!

unsigned char tmp[5];

for ( int i = 0; i < 4; i++) tmp[i] = box_type[i];

tmp[4] = 0;

String^ name = Marshal::PtrToStringAnsi( (IntPtr)  tmp);

return name;

}

Boolean JPEG2000_Manage::close(void)

{

this->filestream->Close();

this->input_file_is_open = false;

return true;

}

Boolean JPEG2000_Manage::is_valid_jp2(void)

{

this->go_to_top_of_file();

// make sure the first box type is jp2/jp2

if(String::Compare(this->get_next_box_header(),"jP  ") == 0 )

{

this->go_to_top_of_file();

return true;

}

return false;

}

void JPEG2000_Manage::go_to_top_of_file(void)

{

if( this->input_file_is_open) this->filestream->Seek(0,SeekOrigin::Begin);

}

Boolean JPEG2000_Manage::skip_box_contents(void)

{

return this->skip_box_contents(this->filestream);

}

Boolean JPEG2000_Manage::skip_box_contents(Stream^ s)

{

// advance to end of box contents

unsigned int new_position;

if( this->box_length > 0 )

{

new_position = s->Position +  this->box_length - 8;

if ( this->xl_box) new_position -= 8; // adjust skip for extra 8 bytes in xl box

if ( new_position >= s->Length)

{

// EOF

s->Position = s->Length ;

return false;

} else {

s->Position = new_position ;

}

return true;

}

else

{

// box extends to end of file - zero length trace

// find current file position

int box_start = s->Position - 8;

s->Position = s->Length; // move to EOF

this->box_length = s->Position - box_start;

return true;

}

}

bool JPEG2000_Manage::fix_zero_length_box()

{

ArrayList^ boxdata = this->get_box_data();

// last box

this->filestream->Position = ((jp2_box_data^)boxdata[boxdata->Count - 1])->position;

if ( ((jp2_box_data^)boxdata[boxdata->Count - 1])->zero_length_box )

{

unsigned int swapped = swap(((jp2_box_data^)boxdata[boxdata->Count - 1])->box_length);

BinaryWriter^ bw = gcnew BinaryWriter(this->filestream);

bw->Write( swapped);

}

// done

return true;

}

bool JPEG2000_Manage::find_box(String^ boxname)

{

if( !this->input_file_is_open ) return false;

if( !this->is_valid_jp2()) return false;

// loop through boxes and box contents

while ( String^ name = this->get_next_box_header())

{

if ( String::Compare(name,boxname) == 0 )

{

this->go_to_top_of_file();

return true;

}

this->skip_box_contents();

if( this->filestream->Position == this->filestream->Length - 1 ) // eof condition

{

this->go_to_top_of_file();

return false;

}

}

return false;

}

// this routine writes an object to a XML text file

Boolean JPEG2000_Manage::write_object_to_xml_text_file(Object^ input_object, String^ xml_text_file_name)

{

XmlSerializer^ ser = gcnew XmlSerializer(input_object->GetType());

XmlTextWriter^ writer = gcnew XmlTextWriter( xml_text_file_name, System::Text::Encoding::UTF8);

ser->Serialize(writer,input_object);

writer->Close();

return true;

}

Object^ JPEG2000_Manage::read_object_from_xml_text_file(System::Type^ input_object_type, String^ xml_text_file_name)

{

Object^ output_object = nullptr;

XmlSerializer^ ser = gcnew XmlSerializer(input_object_type);

XmlTextReader^ reader = gcnew XmlTextReader(xml_text_file_name);

output_object = ser->Deserialize(reader);

reader->Close();

return output_object;

}

Object^  JPEG2000_Manage::read_object_from_xml_gzip_file(System::Type^ output_object_type, String^ xml_gzip_file_name)

{

// we should be a the start of the desired box

// check the next two bytes if they are 1f8b then its a gzip compressed box

Object^ output_object = nullptr;

FileStream^ s = gcnew FileStream(xml_gzip_file_name,System::IO::FileMode::Open);

XmlSerializer^ ser = gcnew XmlSerializer(output_object_type);

GZipStream^ gs = gcnew GZipStream(s,CompressionMode::Decompress);

output_object  =  ser->Deserialize(gs);

gs->Close();

s->Close();

return output_object;

}

System::Void  JPEG2000_Manage::read_object_from_xml_gzip_file(Object^ output_object, String^ xml_gzip_file_name)

{

// we should be a the start of the desired box

// check the next two bytes if they are 1f8b then its a gzip compressed box

FileStream^ s = gcnew FileStream(xml_gzip_file_name,System::IO::FileMode::Open);

XmlSerializer^ ser = gcnew XmlSerializer(output_object->GetType());

GZipStream^ gs = gcnew GZipStream(s,CompressionMode::Decompress);

output_object  =  ser->Deserialize(gs);

return;

}

Boolean JPEG2000_Manage::write_object_to_xml_gzip_file(Object^ input_object, String^ xml_gzip_file_name)

{

FileStream^ fs =  nullptr;

try

{

fs = gcnew FileStream(xml_gzip_file_name,FileMode::Create);

}

catch ( System::Exception^  e)

{

return false;

}

GZipStream ^ compressedzipStream = gcnew GZipStream(fs,CompressionMode::Compress,true );

XmlSerializer^ ser = gcnew XmlSerializer(input_object->GetType());

XmlTextWriter^ writer = gcnew XmlTextWriter( compressedzipStream, System::Text::Encoding::UTF8);

ser->Serialize(writer,input_object);

writer->Close();

compressedzipStream->Close();

fs->Close();

return true;

}

Boolean JPEG2000_Manage::write_object_to_xml_gzip_stream(Object^ input_object, Stream^ ms)

{

GZipStream ^ compressedzipStream = gcnew GZipStream(ms,CompressionMode::Compress,true );

this->write_object_to_xml_stream(input_object,compressedzipStream);

compressedzipStream->Close();

return true;

}

Boolean JPEG2000_Manage::write_object_to_xml_stream(Object^ input_object, Stream^ ms)

{

XmlSerializer^ ser = gcnew XmlSerializer(input_object->GetType());

XmlTextWriter^ writer = gcnew XmlTextWriter( ms, System::Text::Encoding::UTF8);

ser->Serialize(writer,input_object);

return true;

}

Boolean JPEG2000_Manage::write_compressed_xml_box(Object^ input_object, String^ box_label, String^ gzip_xml_box_file_name)

{

FileStream^ ms = gcnew FileStream(gzip_xml_box_file_name,FileMode::Create);

BinaryWriter^ bw = gcnew BinaryWriter(ms);

// write 8 byte preamble to box

bw->Write((int)0);

char* str2 = (char*)(void*)Marshal::StringToHGlobalAnsi(box_label);

for( int i = 0; i < 4; i++) bw->Write((unsigned char) str2[i]);

// write gzipped contents

this->write_object_to_xml_gzip_stream(input_object,ms);

// update length of box

unsigned int ms_size = ms->Length;

ms->Position = 0;

bw->Write(swap(ms_size));

// close up

bw->Close();

return true;

}

Boolean JPEG2000_Manage::write_xml_box(Object^ input_object, String^ box_label, String^ xml_box_file_name)

{

FileStream^ ms = gcnew FileStream(xml_box_file_name,FileMode::Create);

BinaryWriter^ bw = gcnew BinaryWriter(ms);

// write 8 byte preamble to box

bw->Write((int)0);

char* str2 = (char*)(void*)Marshal::StringToHGlobalAnsi(box_label);

for( int i = 0; i < 4; i++) bw->Write((unsigned char) str2[i]);

// write contents .......

this->write_object_to_xml_stream(input_object,ms);

// update length of box

unsigned int ms_size = ms->Length;

ms->Position = 0;

bw->Write(swap(ms_size));

// close up

bw->Close();

return true;

}

void JPEG2000_Manage::Write_Box_Header_to_MemoryStream(String ^box_label, BinaryWriter ^bw, MemoryStream ^ms)

{

// write preamble to stream - note that a memory stream < 2 Gbyte

bw->Write(swap((unsigned int)(ms->Length + 8)));

char* str2 = (char*)(void*)Marshal::StringToHGlobalAnsi(box_label);

for( int i = 0; i < 4; i++) bw->Write((unsigned char) str2[i]);

}

Boolean JPEG2000_Manage::write_xml_box(Object^ input_object, String^ box_label, Stream^ s)

{

BinaryWriter^ bw = gcnew BinaryWriter(s);

MemoryStream^ ms = gcnew MemoryStream();

// put text xml in a memory stream

this->write_object_to_xml_stream(input_object,ms);

Write_Box_Header_to_MemoryStream(box_label, bw, ms);

// dump memory stream to parent stream

ms->WriteTo(s);

return true;

}

Boolean JPEG2000_Manage::write_compressed_xml_box(Object^ input_object, String^ box_label, Stream^ s)

{

BinaryWriter^ bw = gcnew BinaryWriter(s);

MemoryStream^ ms = gcnew MemoryStream();

// put gzip xml in a memory stream

this->write_object_to_xml_gzip_stream(input_object,ms);

Write_Box_Header_to_MemoryStream(box_label, bw, ms);

// dump memory stream to parent stream

ms->WriteTo(s);

return true;

}

ArrayList^ JPEG2000_Manage::get_box_data(void)

{

ArrayList^ r = gcnew ArrayList();

this->go_to_top_of_file();

while(this->filestream->Position < this->filestream->Length)

{

jp2_box_data^ bd = gcnew jp2_box_data();

bd->position = this->filestream->Position;

bd->box_label = this->get_next_box_header();

bd->box_length = this->box_length;

bd->xl_box = this->xl_box;

bd->zero_length_box = this->zero_length_box;

r->Add(bd);

this->skip_box_contents();

}

return r;

}

// Read an xml object from a stream whether compressed or not

Object^ JPEG2000_Manage::read_xml_box(System::Type^ input_object_type, String^ box_label , Stream^ s)

{

Object^ output_object = nullptr;

while ( String::Compare(this->get_next_box_header(s), box_label) != 0 )

{

this->skip_box_contents(s);

if( s->Position >=  s->Length  )return nullptr;

}

// we should be a the start of the desired box

// check the next two bytes if they are 1f8b then its a gzip compressed box

BinaryReader^ br = gcnew BinaryReader(s);

unsigned char c1 = br->ReadByte();

unsigned char c2 = br->ReadByte();

s->Position -= 2; // back up

// read box contents

MemoryStream^ ms ;

if( this->xl_box )

{

ms =  gcnew MemoryStream(br->ReadBytes(this->box_length-16));

} else

{

ms =  gcnew MemoryStream(br->ReadBytes(this->box_length-8));

}

XmlSerializer^ ser = gcnew XmlSerializer(input_object_type);

if ( c1 == 31 && c2 == 139 )

{

// this is gzipped xml

GZipStream^ gs = gcnew GZipStream(ms,CompressionMode::Decompress);

output_object  =  ser->Deserialize(gs);

} else {

output_object  =  ser->Deserialize(ms);

}

return output_object;

}

Boolean JPEG2000_Manage::copy_box(Stream^ is, Stream^ os)

{

// straight copy of one box from one stream to another

// read box header

String^ is_box_label = this->get_next_box_header(is);

if( this->xl_box)

{

is->Position -= 16; // xl box

} else {

is->Position -= 8; // back up

}

if ( is_box_label == nullptr ) return false;

// total length of box

unsigned int is_length = this->box_length;

if( this->box_length == 0 )

{

// this is the last box in the file

is_length = is->Length - is->Position;

}

// assign an io buffer;

unsigned int buffer_length = IO_BUFFER_LENGTH;

if( buffer_length >  is_length ) buffer_length =  is_length ;

array<unsigned char>^ buffer = gcnew array<unsigned char>(buffer_length);

// number of reads

unsigned int nread_full = is_length/buffer_length;

for ( int i = 0; i < nread_full; i++ )

{

is->Read(buffer,0,buffer_length);

os->Write(buffer,0,buffer_length);

}

unsigned int nread_partial = is_length % buffer_length;

if( nread_partial )

{

is->Read(buffer,0,nread_partial);

os->Write(buffer,0,nread_partial);

}

return true;

}

Boolean JPEG2000_Manage::has_compressed_sxml_content(void)

{

unsigned int save_position = this->filestream->Position;

ArrayList^ bd = this->get_box_data(); // get input box info

// look for sxml header

for ( int i = 0; i < bd->Count; i++ )

{

jp2_box_data^ b = (jp2_box_data^)bd[i];

if ( String::Compare(b->box_label,"sxml") == 0 )

{

// found sxml tag

this->filestream->Position = b->position + 8;

if( b->xl_box ) this->filestream->Position += 8; // xl box

// next two characters are gzip magic numbers

unsigned char c1 = this->binaryreader->ReadByte();

unsigned char c2 = this->binaryreader->ReadByte();

if ( c1 == 31 && c2 == 139 )

{

this->filestream->Position = save_position;

return true;

}

}

}

this->filestream->Position = save_position;

return false;

}

Boolean  JPEG2000_Manage::compress_existing_sxml_box(String^ new_file_name)

{

try

{

JPEG2000_Manage^ newjp2 = gcnew JPEG2000_Manage();

newjp2->open(new_file_name); // open a blank file

ArrayList^ bd = this->get_box_data(); // get input box info

this->go_to_top_of_file(); // rewind existing file

for ( int i = 0; i < bd->Count; i ++ ) // cycle through boxes

{

jp2_box_data^ bdi = (::jp2_box_data^) bd[i];

if ( String::Compare(bdi->box_label,"sxml") == 0 )

{

JPEG2000_SEGY^ jp2sgy = gcnew JPEG2000_SEGY() ;

jp2sgy = (JPEG2000_SEGY^) this->read_xml_box(jp2sgy->GetType(),"sxml",this->filestream);

//rewrite in a compressed format

this->write_compressed_xml_box(jp2sgy,"sxml",newjp2->filestream);

} else {

this->copy_box(this->filestream, newjp2->filestream);

}

}

// insure output file has not zero length boxes

newjp2->fix_zero_length_box();

newjp2->close();

}

catch (  System::Exception^ )

{

return false;

}

return true;

}

## Appendix 2: What is so special about JPEG 2000

JPEG 2000 should definitely not be confused with the commonly-used file format JPEG. It represents a total reengineering of the way images and data can be packaged, viewed,  annotated, and disseminated over Internet channels.

Although it is outside the bounds of this document to fully discuss this framework, some of the important and salient points of this approach to image storage of compression are listed below:

- Open file standard ISO/IEC 15444-1:2000

- Wavelet based, multiresolution representation

- Up to 38 bit signed data – not just images

- Up to 16,000 planes/channels

- Entropy-based (MQ) bit-plane encoding (save 20 bits instead of 32, white space costs almost nothing)

- Lossless/lossy encoding - harmonic distortion for lossy compression

- Flexible file format –XML-aware,  UUID defined boxes

- Random access to ROI, transcoding, quality layers,etc

- Internet ready : JPIP => low bandwidth optimized

- Industry support: e.g., Lizardtech, Adobe Photoshop

- Custom wavelet transforms