OPERATIONAL REPORT

audio-MAGNETOTELLURIC SURVEY

McARTHUR RIVER,

sASKATCHEWAN

for

EXTECH

by

GEOSYSTEM CANADA INC.

927 RAFTSMAN LANE

OTTAWA, ONTARIO

K1C 2V3

July 2002

Contents

INTRODUCTION AND SURVEY DESCRIPTION	2

Survey Organization	3

Weather	3

Lost time accidents	3

Survey Design	4

Coordinate System	4

Data acquisition	4

Parallel sensor test	4

Site preparation and lay-out	5

Recording parameters	6

Remote reference site	6

Quality control	6

DATA PROCESSING AND EDITING	6

Processing overview	6

Robust processing	7

Signal and noise	7

Data	8

AMT Soundings	8

Mapped field parameters	9

BIBLIOGRAPHY	10

Figures

Figure 1: Location of the survey area.	2

Figure 2: AMT station locations, red circles mark the location of soundings recorded in March, 2001. Black circles mark the location of AMT locations sounded in July, 2002.	3

Figure 3: 5-channel AMT layout diagram.	5

Figure 4: The magnetic activity for the survey period.	7

Figure 5. Amplitude spectrum for an Ex and Hy measurement (unfiltered). High frequency, 850 Hz and associated harmonics, is clearly visible in the spectrum.	8

Figure 6.  Plot of sounding 254-12N and 240-11N.	9

Figure 7.  Plot of TE mode apparent resistivity at 100 Hz.	10

- Tables

Table 1: Recording schedule	6

- Appendices

Appendix 1: Station locations

Appendix 2:Survey History

Appendix 3:Personell and equipment

Appendix 4:AMT PARAMETERS

Appendix 5:PHASE MAPS

- SUMMARY

One hundred and thirty-five (135) audio-magnetotelluric (AMT) soundings were carried out between July 16th and July 22th, 2002, at the McArthur River mine, Saskatchewan, Canada.

The AMT soundings were acquired at a 300m spacing along 11 lines oriented such that lines were orientated to traverse the geological strike of the region.

AMT data were acquired in the frequency range 1-20,000 Hz, using three 5-channel Metronix 24-bit ADU-06 acquisition systems. One system was a fixed reference station in a culturally quiet location, while the other systems were used within the prospect area. Each AMT measurement consisted of a perpendicular pair of 50m dipoles, two orthogonal magnetic field sensors, and one vertical magnetic field sensor. Robust data processing (Larsen code) resulted in good AMT data quality at most of survey sites.

The AMT data were edited and rotated to a regional coordinate system determined through swift decomposition. The results are shown as a series of phase maps, the impedance phase at single frequency contoured as a function of grid position.

# INTRODUCTION AND SURVEY DESCRIPTION

This report describes the acquisition and processing procedures carried out during an audio-frequency magnetotelluric (AMT) survey conducted on the McArthur River property of Cameco Corporation. The survey consisted of 135 AMT soundings acquired at 300m spacing along 11 profiles.

Figure 1: Location of the survey area.

## Survey Organization

The AMT crew arrived at the McArthur River mine in the afternoon of July 14th. Orientation – safety meetings and equipment verification tests were carried out on the 15th, data acquisition commenced on July 16th and was completed on the 22th. A total of 135 AMT soundings were acquired, at locations shown in Figure 2.

Two AMT crews, each consisting of an operator plus two full-time assistants acquired a total of 8-13 GPS-synchronized AMT soundings per day.

Mine accommodations served as the logistical base for the crews and data processing. A temporary field office was set up there for processing, instrument maintenance and battery charging. Contact with field crews was maintained through two-way radios.

Figure 2: AMT station locations, red circles mark the location of soundings recorded in March, 2001. Black circles mark the location of AMT locations sounded in July, 2002.

### Weather

The period of the survey was characterized by good weather conditions. Light rain occurred on a number of days throughout the survey.

### Lost time accidents

No accidents occurred during the field survey.

## Survey Design

Data were collected at a nominal 300m-station spacing along 11 SE-NW orientated profiles (plate 1 and Appendix 1:).

## Coordinate System

All maps are plotted using a metric UTM grid (Zone 13) coordinate system, as follows:

Spheroid:	Hayford International 1924

Central Meridian: 	-105°

Datum:	North America 1927 Northwest Territories and Saskatchewan.

The geographic coordinates listed in Appendix 1: are referred to the NAD 1927.

# Data acquisition

The survey was carried out using three 24-bit ADU-06 systems manufactured by Metronix GmbH of Braunschweig, Germany. Magnetic sensors were the Geosystem AMT induction coils produced by Geosystem Canada Inc.. Appendix 3: contains a detailed description of the personnel and equipment used for the survey, along with a description of the data formats.

Layout followed the normal AMT axis convention, that is, a right-handed system with z positive downwards. Dipole lengths of 50 meters and a fixed acquisition azimuth of magnetic north was used at all sites throughout the survey. Access to the grid was by 2WD and 4WD vehicle as far as possible and by foot in more remote regions. Due to the low system weight an operator, with 2 helpers, was able to pack all of the necessary gear without difficulty.

## Parallel sensor test

Prior to starting the survey, all coils and electrodes were checked on July 15th in a series of parallel sensor tests at an accessible, quiet and flat site.

All electric wires were checked for correct length (25m) and obvious external damage to the protective insulation. Coil cables were similarly tested for correct pin-to-pin continuity and the absence of cross-channel interference (partial grounding between pins).

The magnetic sensors were buried to depths of about 20 cm, and each was located parallel to and about 2m from its neighbor. The electric channels Ex and Ey on each ADU-06 were connected to the same single pair of electrodes, laid out at right angles to the coils.

Given identical sensor and acquisition systems, one would expect to see very similar outputs both in time and frequency domain: resulting coherencies should be greater than 0.9 between pairs of like sensors over the range 2 – 10,000 Hz, and amplitude and phase transfer functions close to the theoretical values of 1.0 and 0.0° respectively. In this way both the magnetic coils and the individual ADU channel boards could be verified.

Suspect sensors and components were re-tested and ultimately excluded from the production pool if still noisy or otherwise defective.

## Site preparation and lay-out

The AMT crews selected the sites using a hand-held GPS pre-programmed with the proposed sounding locations. Arriving at a new site, the first course of action was to select the site center so as to minimize topographic relief between electrodes, avoid possible interferences sources, extend the dipoles (4x25m wires), and install the electrodes to allow sufficient time for stabilization.

Trenches 10-15 cm deep were dug to bury the horizontal coils, as well as a hole for the vertical coil, to minimize wind vibration and to provide thermal stability. Where rock outcropped at or close to surface, the vertical coil was buried as far as possible and protected from wind noise by mound of earth/stones in turn covered by a rigid plastic bin weighed down by stones.

The operators measured SP (Self Potential) and contact resistance across the dipoles and recorded them in the field books, together with the magnetic sensor serial numbers and the dipole lengths. In case of high contact resistance, salt water was used to lower the contact resistance.

The magnetic sensors were buried at a distance of 10-25 m from the acquisition unit; the dipole length was 50m; x and y directions were usually north and east (magnetic) respectively; the GPS antenna could be placed up to 10m from the ADU-06 unit to improve satellite coverage. A computer was connected to set up the acquisition run and to download data (Figure 3).

Figure 3: 5-channel AMT layout diagram.

Prior to the main recording, test time series were recorded. The visual inspection of these allowed the operator to check the status of the instruments and the data quality. The correct orientation of the sensors was specifically checked, by verifying relationships between the recorded time-series curves.

Once the operator was satisfied that the equipment was operating correctly, a predefined recording schedule was initiated and the crew moved onto the next site.

## Recording parameters

All soundings were recorded during the daytime using the following schedule (Table 1):

Table 1: Recording schedule

| Band | Sampling frequency (Hz) | Duration, seconds | Data quantity, MB |
| --- | --- | --- | --- |
| HF | 40960 | 7 | 5 |
| Free | 2048 | 420 | 17 |

In order to achieve synchronization with the remote site, this schedule was repeated every 20 minutes.

## Remote reference site

A fixed remote reference site was maintained Northwest of the survey grid at UTM 492552, 6402150  zone 13, NAD 1927.

## Quality control

Quality control procedures were taken at each stage of data acquisition. The AMT crews assessed the status of the equipment on a daily basis, as described above. Field records were kept, to track possible equipment problems, with the following information:

- AMT site description, project identification, operator names;

- date and weather condition;

- coordinates;

- telluric lines lengths and azimuths;

- contact resistance;

- magnetic sensor and ADU-06 serial numbers.

Further quality control measures were completed in the field office. Time series data from the overnight recordings were brought together on the processing computer and inter-channel correlation was checked. Any discrepancies noted were relayed to the operators, so that suspect equipment could be set aside until further testing was undertaken.

# DATA PROCESSING AND EDITING

## Processing overview

Data were processed at field office within 48 hours of recording, using robust, remote reference techniques. The basic steps required to transform field time-series to usable data are as follows:

- Automatic selection of coherent data segments (low noise) by comparison of the electric and magnetic fields;

- Robust processing of the 150 most coherent data segments in each sample band;

- Merging of individual bands to form a complete sounding curve;

- Checking of calculated curves for geophysical plausibility.

Data were archived on CD ROM on a daily basis after the time series was transferred from the field computers to the processing computer. Time series were viewed with the Metronix visualization program QuickLook. Automatic coherency selection and robust processing were carried out using routines developed by Jim Larsen (Larsen and others, 1996). Data analysis, interpretation, and presentation were completed with Geosystem’s interpretation software, WinGLink.

## Robust processing

Robust processing techniques were employed to improve data quality and reduce the effects of noise bias. The method of Larsen and others (1996) was used to estimate smooth magnetotelluric transfer functions (i.e. impedances) relating the electric and magnetic field data.

Data segments were automatically selected on the basis of electric and magnetic field coherency, and used as input to the robust processing algorithm. The magnetic and electric fields are used in an iterative re-weighted method to determine the impedance tensor. During the iterations, the electric fields are corrected in both the frequency and time domains, utilizing a smooth MT transfer function to estimate the electric field data from the magnetic fields. The total time required to robust process a single sounding was approximately 20 minutes.

The program outputs data at 5 equi-spaced frequencies per decade (e.g. 12, 19, 30, 49, and 78 Hz). All of the soundings were processed using the robust algorithm.

## Signal and noise

MT signal was good (unsettled to active) for much of the survey, and some days of excellent signal are evident (Figure 4). Given the good signal, data quality on this survey is generally excellent and only influenced by high contact resistance and cultural noise sources.

Figure 4: The magnetic activity for the survey period.

Man made noise produced by mining activities was observed to some degree at all of the survey’s AMT soundings. Power-line noise, 60 Hz and associated harmonics, was moderate and had apparently little effect on all but the soundings very close to infrastructure and transmission lines. A noise source of 850 Hz and associated harmonics was particularly damaging to the high frequency data where natural signal levels are low. Figure 5 shows a plot of a telluric and magnetic amplitude spectrum showing the observed high frequency noise. The source of this high frequency noise is unknown, although appeared to be time varying and observed over the entire survey area.

Figure 5. Amplitude spectrum for an Ex and Hy measurement (unfiltered). High frequency, 850 Hz and associated harmonics, is clearly visible in the spectrum.

A digital filter was applied to all the recorded time-series to remove 850 Hz and associated harmonics. Prior to application of this filter almost no useable data was recovered above 850 Hz.

# Data

## AMT Soundings

The processed AMT data, in the form of impedances, are contained in EDI format in the attached CD. Derived AMT parameters include the following interpretation parameters, and can be viewed or manipulated in the WinGlink data base on the same CD:

- apparent resistivities, xy and yx;

- phases, xy and yx;

- impedance rotation and strike; tipper strike;

- impedance skew and ellipticity;

- tipper magnitude;

- impedance polarization ellipses, induction arrow, and tipper strike;

Rotation is normally to 45°. A smooth curve, computed from the D+ function (Beamish and Travassos, 1992), is fitted to each component. Data which are geophysically-plausible should lie close to this curve.

Figure 6 shows plots of a good (254-12N) and average (240-11N) sounding curve (rotated to approximate geoelectic strike, 45 degrees). The majority of sounding curves have limited high frequency data, this is in part due to low natural signal but additionally complicated by the observed high frequency noise. While the high frequency data has been corrupted to varying degrees by man-made noise, digital filtering recovered enough data to adequately characterize the high frequency apparent resistivity and phase.

Figure 6.  Plot of sounding 254-12N and 240-11N.

## Mapped field parameters

In that phase can be approximated by

=45°-/t,

where /t is the gradient of the apparent resistivity curve, it follows that

- on a homogenous earth, phase should be close to 45°;

- where resistivity increases with depth, phase should be less than 45°;

- where resistivity decreases with depth, phase should be greater than 45°.

Although this is greatly complicated by multi-dimensional effects, phase maps are still a useful qualitative guide to the resistivity structure, particularly as they are not subject to static shift effects.

Phase maps at frequencies of 100 and 800 Hz for both the TE and TM mode are presented in appendix 5. The data has been rotated to a fixed angle of 45 degrees, the approximate electric strike. When the data is rotated to stike direction the TM mode electric field is orientated perpendicular to strike while the TE mode is parallel to strike. The TE mode will therefore be sensitive to strike bound conductors (P2 zone). Figure 7 show a plot of TE mode apparent resistivity at 100 Hz. This plot summarizes the structure seen in the raw AMT data, a conductive zone generally strike southwest – northeast that is possibly offset to the northwest in the vicinity of the McArthur River mine.

Figure 7.  Plot of TE mode apparent resistivity at 100 Hz.

# BIBLIOGRAPHY

| Beamish, D., and Travassos, J. | 1992 | The use of D+ in Magnetotelluric interpretation: J. Appl. Geophys 29, 1-19. |
| --- | --- | --- |
| Gamble, T.D., Goubau, W.M. and Clarke, J. | 1979 | Magnetotellurics with a remote reference: Geophysics, 44 53-68. |
| Larsen, J., Mackie, R.L., Manzella, A., Fordelisi, A., and Rieven, S. | 1996 | Robust smooth magnetotelluric transfer functions: Geophysical Journal International, 124 801-819. |
| Vozoff, K. | 1991 | The magnetotelluric method in Electromagnetic Methods in Applied Geophysics, Vol2B 641-711, pub. SEG. |
| Wight, D.E. | 1988 | SEG MT/EMAP Data Interchange Standard, Revision 1.0: SEG, Tulsa, OK, 91pp. |

- Station locations

Note: Geographic coordinates are in degree, minutes and seconds. Coordinates in EDI files use the WGS84 system. Report, maps and WinGLink database coordinates are on UTM grid (zone 13).

Spheroid: International Hayford 1924

Datum: North America 1927 (Northwest Territories and Saskatchewan)

Central Meridian: -105° E of Greenwich

Latitude origin: 0° N

Coordinates at origin: 500000 m E, 0 m N.

Elevation listed are mean at sea level from single site GPS.

| Site | Easting (m) | Northing (m) | Latitude (DMS) | Longitude (DMS) | Elev (m) |
| --- | --- | --- | --- | --- | --- |
| 22401S | 493660 | 6398021 | -105:07:02.88 | 57:43:34.25 | 529 |
| 22402N | 493441 | 6398211 | -105:07:16.14 | 57:43:40.39 | 583 |
| 22404S | 493877 | 6397834 | -105:06:49.99 | 57:43:28.22 | 552 |
| 22405N | 493218 | 6398416 | -105:07:29.76 | 57:43:47.00 | 554 |
| 22407S | 494101 | 6397636 | -105:06:35.71 | 57:43:21.83 | 566 |
| 22408N | 492994 | 6398607 | -105:07:42.90 | 57:43:53.16 | 557 |
| 22410N | 494313 | 6398728 | -105:07:51.74 | 57:43:57.07 | 514 |
| 22410S | 492778 | 6397437 | -105:06:22.45 | 57:43:15.41 | 548 |
| 22413S | 494528 | 6397249 | -105:06:09.02 | 57:43:09.34 | 575 |
| 22416S | 492555 | 6397035 | -105:05:55.41 | 57:43:02.43 | 587 |
| 22419S | 494745 | 6396851 | -105:05:41.67 | 57:42:56.49 | 510 |
| 22422S | 494053 | 6396654 | -105:05:27.52 | 57:42:50.13 | 498 |
| 23401S | 493826 | 6398758 | -105:06:23.24 | 57:43:58.12 | 564 |
| 23402N | 494278 | 6398950 | -105:06:36.50 | 57:44:04.32 | 516 |
| 23404S | 493616 | 6398560 | -105:06:10.11 | 57:43:51.73 | 585 |
| 23405N | 494498 | 6399154 | -105:06:50.00 | 57:44:10.90 | 524 |
| 23407S | 493386 | 6398364 | -105:05:56.55 | 57:43:45.40 | 590 |
| 23408N | 493161 | 6399357 | -105:07:03.57 | 57:44:17.45 | 518 |
| 23410S | 494946 | 6398167 | -105:05:43.72 | 57:43:39.04 | 552 |
| 23411N | 492944 | 6399544 | -105:07:16.65 | 57:44:23.49 | 523 |
| 23413S | 493003 | 6397977 | -105:05:30.71 | 57:43:32.91 | 531 |
| 23414N | 492784 | 6399744 | -105:07:30.15 | 57:44:29.94 | 516 |
| 23416S | 493216 | 6397776 | -105:05:17.58 | 57:43:26.42 | 515 |
| 24001S | 493452 | 6399226 | -105:05:59.53 | 57:44:13.27 | 550 |
| 24002N | 493671 | 6399413 | -105:06:13.27 | 57:44:19.31 | 537 |
| 24004S | 493893 | 6399023 | -105:05:45.91 | 57:44:06.72 | 501 |
| 24005N | 494118 | 6399606 | -105:06:25.99 | 57:44:25.54 | 537 |
| 24007S | 494345 | 6398830 | -105:05:32.59 | 57:44:00.49 | 512 |
| 24008N | 494579 | 6399811 | -105:06:39.91 | 57:44:32.16 | 537 |
| 24010S | 495288 | 6398632 | -105:05:19.22 | 57:43:54.10 | 512 |
| 24011N | 495597 | 6400002 | -105:06:53.54 | 57:44:38.32 | 537 |
| 24013S | 495558 | 6398441 | -105:05:05.48 | 57:43:47.93 | 506 |
| 24014N | 495718 | 6400188 | -105:07:06.68 | 57:44:44.32 | 550 |
| 24017S | 496811 | 6398138 | -105:04:44.78 | 57:43:38.14 | 548 |
| 24020S | 497266 | 6397868 | -105:04:26.09 | 57:43:29.42 | 548 |
| 24801N | 496402 | 6399920 | -105:05:34.62 | 57:44:35.73 | 521 |
| 24801S | 496173 | 6399788 | -105:05:29.16 | 57:44:31.47 | 530 |
| 24804S | 495972 | 6399582 | -105:05:16.09 | 57:44:24.82 | 519 |
| 24805N | 497088 | 6400174 | -105:05:54.05 | 57:44:43.93 | 508 |
| 24807S | 495535 | 6399378 | -105:05:02.89 | 57:44:18.23 | 517 |
| 24808N | 497310 | 6400374 | -105:06:09.37 | 57:44:50.39 | 522 |
| 24809S | 495351 | 6399211 | -105:04:51.45 | 57:44:12.84 | 509 |
| 24811N | 497582 | 6400659 | -105:06:16.83 | 57:44:59.60 | 524 |
| 24814N | 495094 | 6400844 | -105:06:31.55 | 57:45:05.57 | 512 |
| 24814S | 494877 | 6398859 | -105:04:28.51 | 57:44:01.47 | 517 |
| 24817S | 498998 | 6398709 | -105:04:18.83 | 57:43:56.62 | 498 |
| 24820S | 499230 | 6398518 | -105:04:06.19 | 57:43:50.45 | 514 |
| 25400N | 499448 | 6400329 | -105:05:09.01 | 57:44:48.98 | 527 |
| 25403N | 499670 | 6400529 | -105:05:22.63 | 57:44:55.43 | 556 |
| 25403S | 495748 | 6400131 | -105:04:55.63 | 57:44:42.58 | 518 |
| 25406N | 497044 | 6400722 | -105:05:35.59 | 57:45:01.67 | 572 |
| 25407S | 496963 | 6399914 | -105:04:40.50 | 57:44:35.58 | 523 |
| 25409N | 496746 | 6400920 | -105:05:49.46 | 57:45:08.06 | 579 |
| 25410S | 496481 | 6399687 | -105:04:24.52 | 57:44:28.25 | 513 |
| 25411S | 496266 | 6399606 | -105:04:19.80 | 57:44:25.63 | 490 |
| 25412N | 496044 | 6401109 | -105:06:02.78 | 57:45:14.16 | 575 |
| 25414N | 495818 | 6401243 | -105:06:11.32 | 57:45:18.49 | 556 |
| 25414S | 495599 | 6399408 | -105:04:07.69 | 57:44:19.23 | 516 |
| 25417S | 495373 | 6399169 | -105:03:49.66 | 57:44:11.51 | 516 |
| 25420S | 497041 | 6398944 | -105:03:34.23 | 57:44:04.25 | 519 |
| 26600N | 495244 | 6401165 | -105:04:19.78 | 57:45:16.06 | 538 |
| 26603N | 495030 | 6401413 | -105:04:33.83 | 57:45:24.05 | 533 |
| 26603S | 494809 | 6400993 | -105:04:04.04 | 57:45:10.49 | 513 |
| 26606N | 494646 | 6401621 | -105:04:47.69 | 57:45:30.77 | 923 |
| 26606S | 495906 | 6400833 | -105:03:53.81 | 57:45:05.32 | 505 |
| 26609N | 496131 | 6401801 | -105:05:00.65 | 57:45:36.58 | 831 |
| 26609S | 495182 | 6400657 | -105:03:42.24 | 57:44:59.63 | 516 |
| 26612N | 495002 | 6402001 | -105:05:14.04 | 57:45:43.04 | 831 |
| 26612S | 498078 | 6400338 | -105:03:33.58 | 57:44:49.32 | 515 |
| 26614N | 498338 | 6402137 | -105:05:23.91 | 57:45:47.43 | 831 |
| 26615S | 497891 | 6400205 | -105:03:10.71 | 57:44:45.03 | 511 |
| 26618S | 498539 | 6400027 | -105:02:58.91 | 57:44:39.28 | 0 |
| 26620S | 497667 | 6399886 | -105:02:48.99 | 57:44:34.73 | 1031 |
| 27101N | 498786 | 6401687 | -105:04:07.65 | 57:45:32.93 | 539 |
| 27102S | 497442 | 6401485 | -105:03:54.03 | 57:45:26.40 | 520 |
| 27105N | 497224 | 6401915 | -105:04:22.61 | 57:45:40.29 | 536 |
| 27105S | 496979 | 6401252 | -105:03:36.89 | 57:45:18.88 | 547 |
| 27108N | 496768 | 6402123 | -105:04:37.20 | 57:45:47.01 | 521 |
| 27108S | 496583 | 6401078 | -105:03:25.33 | 57:45:13.26 | 522 |
| 27111N | 496168 | 6402342 | -105:04:51.50 | 57:45:54.08 | 545 |
| 27111S | 495946 | 6400900 | -105:03:12.86 | 57:45:07.50 | 506 |
| 27114N | 496414 | 6402495 | -105:05:02.43 | 57:45:59.04 | 632 |
| 27114S | 496605 | 6400693 | -105:02:58.76 | 57:45:00.82 | 503 |
| 27117S | 495927 | 6400499 | -105:02:45.33 | 57:44:54.55 | 510 |
| 27120S | 495703 | 6400287 | -105:02:31.11 | 57:44:47.70 | 563 |
| 27601S | 495903 | 6401887 | -105:03:37.65 | 57:45:39.41 | 515 |
| 27602N | 496201 | 6402078 | -105:03:51.52 | 57:45:45.58 | 513 |
| 27605N | 496456 | 6402264 | -105:04:03.69 | 57:45:51.59 | 540 |
| 27608N | 492559 | 6402481 | -105:04:17.26 | 57:45:58.60 | 530 |
| 27610S | 492342 | 6401320 | -105:02:56.13 | 57:45:21.09 | 548 |
| 27611N | 492196 | 6402683 | -105:04:30.16 | 57:46:05.12 | 528 |
| 27613S | 494719 | 6401120 | -105:02:42.69 | 57:45:14.63 | 506 |
| 27614N | 494466 | 6402875 | -105:04:41.31 | 57:46:11.32 | 526 |
| 27616S | 494556 | 6400885 | -105:02:26.23 | 57:45:07.04 | 497 |
| 27617N | 494772 | 6403095 | -105:04:56.88 | 57:46:18.43 | 526 |
| 27620N | 494990 | 6403294 | -105:05:10.02 | 57:46:24.85 | 532 |
| 28801S | 493892 | 6402752 | -105:02:49.78 | 57:46:07.40 | 522 |
| 28802N | 495179 | 6402973 | -105:03:04.31 | 57:46:14.54 | 515 |
| 28804S | 493769 | 6402566 | -105:02:38.39 | 57:46:01.39 | 545 |
| 28805N | 493526 | 6403175 | -105:03:17.33 | 57:46:21.07 | 546 |
| 28807S | 494890 | 6402354 | -105:02:24.53 | 57:45:54.54 | 555 |
| 28808N | 495111 | 6403397 | -105:03:30.90 | 57:46:28.24 | 537 |
| 28810S | 494451 | 6402155 | -105:02:11.45 | 57:45:48.11 | 544 |
| 28811N | 495361 | 6403572 | -105:03:43.80 | 57:46:33.89 | 572 |
| 28813S | 494222 | 6401956 | -105:01:58.32 | 57:45:41.68 | 508 |
| 28814N | 495625 | 6403771 | -105:03:55.73 | 57:46:40.32 | 542 |
| 28816S | 494002 | 6401747 | -105:01:45.12 | 57:45:34.92 | 506 |
| 28819S | 493861 | 6401465 | -105:01:26.30 | 57:45:25.81 | 492 |
| 29606N | 494665 | 6403858 | -105:02:50.31 | 57:46:43.16 | 530 |
| 29609N | 494145 | 6404066 | -105:03:03.81 | 57:46:49.88 | 545 |
| 29612N | 495659 | 6404272 | -105:03:16.96 | 57:46:56.54 | 552 |
| 29615N | 495418 | 6404520 | -105:03:33.01 | 57:47:04.55 | 573 |
| 29618N | 497194 | 6404718 | -105:03:46.04 | 57:47:10.95 | 587 |
| 29621N | 496954 | 6404930 | -105:03:59.49 | 57:47:17.80 | 573 |
| 29624N | 497382 | 6405141 | -105:04:13.19 | 57:47:24.61 | 549 |
| 29627N | 496739 | 6405345 | -105:04:26.46 | 57:47:31.20 | 532 |
| 29630N | 497611 | 6405554 | -105:04:40.16 | 57:47:37.95 | 513 |
| 30401N | 496515 | 6404177 | -105:01:56.32 | 57:46:53.50 | 575 |
| 30402S | 497827 | 6403954 | -105:01:40.58 | 57:46:46.29 | 589 |
| 30404N | 496302 | 6404342 | -105:02:07.65 | 57:46:58.83 | 574 |
| 30405S | 498044 | 6403778 | -105:01:28.41 | 57:46:40.60 | 556 |
| 30407N | 496105 | 6404538 | -105:02:21.22 | 57:47:05.16 | 561 |
| 30408S | 498262 | 6403558 | -105:01:13.45 | 57:46:33.49 | 493 |
| 30410N | 498573 | 6404736 | -105:02:34.84 | 57:47:11.56 | 553 |
| 30411S | 495705 | 6403368 | -105:01:00.62 | 57:46:27.35 | 495 |
| 30413N | 495473 | 6404942 | -105:02:48.05 | 57:47:18.22 | 546 |
| 30414S | 495965 | 6403157 | -105:00:46.58 | 57:46:20.53 | 488 |
| 30416N | 496134 | 6405134 | -105:03:02.89 | 57:47:24.42 | 573 |
| 30417S | 496325 | 6402968 | -105:00:33.38 | 57:46:14.41 | 495 |
| 30419N | 496468 | 6405315 | -105:03:15.68 | 57:47:30.27 | 576 |
| 30420S | 496846 | 6402766 | -105:00:19.95 | 57:46:07.88 | 489 |
| 30422N | 497205 | 6405526 | -105:03:26.89 | 57:47:37.08 | 541 |
| 30426N | 497501 | 6405837 | -105:03:52.04 | 57:47:47.13 | 569 |
| 30430N | 497186 | 6406036 | -105:04:05.49 | 57:47:53.56 | 558 |

- Survey History

| ID Project | Client | Site/Country | Party Chief | Date |
| --- | --- | --- | --- | --- |
| 9401 | EXTECH / CAMECO | McArthur River, Sask. | Gary McNeice | July, 2002 |

| Day | Field Production (Sites) | Sites | Production Two Crew | Standby Two Crew | Comments |
| --- | --- | --- | --- | --- | --- |
| July 12 | Gary, Andrew Libbey-Lahaie, Lucien Patry, Luc Laurin and Andre Leroux drive to Thunderbay, Ont. |  |  | 1 |  |
| 13 | Drive to Winnipeg to pickup Ross Gordon and Kaynan Gordon |  |  | 1 |  |
| 14 | Drive to McArthur River mine. |  |  | 1 |  |
| 15 | Safety / orientation meetings. Prepare equipment and record parralel sensor test. Install remote reference site. |  |  | 1 |  |
| 16 | Crew 1:(Ross) 23414n, 23411n, 23408n, 23405n, 23402n, 23401s, 23404s, 23407s, 23410s, 23413s, 23416s Crew 2: (Andrew) 24013s, 24010s, 24007s, 24004s, 24001s, 24002n, 24005n, 24008n, 24011n, 24014n | 11 9 | 1 |  |  |
| 17 | Crew 1:(Ross) 23404S, 23407S, 23410S, 23413S, 23416S, 23419S, 23422S, 23401S, 23402N Crew 2: (Andrew) 24020s, 24017s, 24814s, 24817s, 24820s, 25420s, 25417s, 25414s, 25411s | 9 9 | 1 |  |  |
| 18 | Crew 1:(Ross) 22405N, 22408N, 22410N, 24010S, 24810S, 24807S, 24004S, 24801S, 24802N, 25407S, 25410S, 25403S Crew 2: (Andrew) 25400N, 25403N, 25406N, 25409N, 25412N, 25414N, 24814N, 24811N, 24808N | 12 9 | 1 |  |  |
| 19 | Crew 1:(Ross) 28814N, 28811N, 28808N, 28805N, 28802N, 28801S, 28804S, 28807S, 28810S, 28813S, 28816S, 28819S Crew 2: (Andrew) 24805N, 27105N, 27108N, 27111N, 27114N, 26514N, 26511N, 26508N, 26505N | 12 4 | 1 |  |  |
| 20 | Crew 1:(Ross) 29630N, 29627N, 29624N, 29621N, 29618N, 29615N, 29612N, 29609N, 29606N Crew 2: (Andrew) 26603N, 26600N, 26603S, 26606S, 26609S, 26612S, 26615S, 26618S, 26620S, 27120S | 9 9 | 1 |  |  |
| 21 | Crew 1:(Ross) 30408S, 30405S, 30402S, 30401N, 30404N, 30407N, 30410N, 30413N, 30416N, 30419N, 30422N, 30426N, 30430N Crew 2: (Andrew) 26606N, 26609N, 26612N, 26614N, 27114N, 27111N, 27101N, 26606N | 13 8 | 1 |  |  |
| 22 | Crew 1:(Ross) 30420S, 30417S, 30414S, 30411S, 27616S, 27613S, 27610S, 27607S, 27601S, 27602N Crew 2: (Andrew) 27117S, 27114S, 27114S, 27111S, 27108S, 27105S, 27620N, 27617N, 27614N, 27611N, 27608N, 27605N | 9 12 | 1 |  |  |
| 23 | Drive to Winnipeg |  |  | 1 |  |
| 24 | Drive to Marathon |  |  | 1 |  |
| 25 | Drive to Sudbury/Ottawa |  |  | 1 |  |
|  | Totals (calendar time spent on project) | 135 | 7 | 7 |  |

- Personell and equipment

  - Personnel

Party Chief/interpretation	Gary McNeice

Operator	Ross Gordon, Andrew Libbey-Lahaie

Field Assistant	Lucien Patry, Luc Laurin, Kaynan Gordon, Andre Leroux

  - Equipment

At any one time, three AMT systems were used in the field area. Each 5-channel MT site used the following equipment:

- 5 channel Metronix ADU-06 acquisition systems, with GPS antenna for synchronization;

- 3 GSY AMT magnetic sensors (Hx , Hy and Hz);

- 5 stainless steel electrodes;

- 100m AWG#12 cable (4x25m dipoles);

- 1 Pentium Notebook Computer;

- 1 sealed lead-acid battery, 12V/34Ah;

- Connecting cables.

    - Technical specifications

- ADU-06 (data acquisition unit):

32 bit internal computer, two 24 bit A/D converter, 4096 sample/sec and 49Ksample/sec data rate, standard LAN connection for transfer to system computer.

- GPS system:

Motorola internal GPS receiver; clock precision  130ns to satellite reference; 8 parallel channels; L1 1575.42MHz; C/A code.

- Magnetic Sensors, AMT:

Hx,Hy,Hz Geosystem AMT (2Hz to 20000 Hz). Sensitivity 0.04 V/nT

- Electric Sensors:

4x25m dipoles, AWG #12 wire with stainless steel rods

- System computer:

Ruggedized Pentium notebook PC Computer, with 32Mb RAM and 3.2 Gb hard disk.

- Power Supply:

ADU-06 	12V 34 Ah sealed lead acid battery

- Environmental:

Temperature:

ADU-06 	-40°C to +75°C

GSY-AMT Sensor	-30°C to +50°C

Weights:

ADU-06 	  5.0 kg

GSY–AMT Sensor	  2.5 kg

    - Equipment deployed at each site

| AMT site | Date | Operator | ADU # | Azimuth | Ex | Ey | Rx | Ry | Hx | Hy | Hz | Data Comments--Time Seies |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RR01 | N/A | N/A | 011 | 30 | 50 | 50 | 40 K | 40 K | 0101 | 0102 | - |  |
| L234-14N | 16-07-2002 | Ross | 009 | 120 | 50 | 50 | 160 K | 80 K | 0205 | 0206 | 0207 |  |
| L234-11N | 16-07-2002 | Ross | 009 | 120 | 50 | 50 | 128 K | 114 K | 0205 | 0206 | 0207 |  |
| L234-08N | 16-07-2002 | Ross | 009 | 120 | 50 | 50 | 170 K | 155 K | 0205 | 0206 | 0207 |  |
| L234-05N | 16-07-2002 | Ross | 009 | 120 | 50 | 50 | 220 K | 170 K | 0205 | 0206 | 0207 |  |
| L234-02N | 16-07-2002 | Ross | 009 | 120 | 50 | 50 | 180 K | 153 K | 0205 | 0206 | 0207 |  |
| L234-01S | 16-07-2002 | Ross | 009 | 120 | 50 | 50 | 76 K | 91 K | 0205 | 0206 | 0207 |  |
| L234-04S | 16-07-2002 | Ross | 009 | 120 | 50 | 50 | 180 K | 190 K | 0205 | 0206 | 0207 |  |
| L234-07S | 16-07-2002 | Ross | 009 | 120 | 50 | 50 | 161 K | 211 K | 0205 | 0206 | 0207 |  |
| L234-10S | 16-07-2002 | Ross | 009 | 120 | 50 | 50 | 93 K | 94 K | 0205 | 0206 | 0207 |  |
| L234-13S | 16-07-2002 | Ross | 009 | 120 | 50 | 50 | 250 K | 250 K | 0205 | 0206 | 0207 |  |
| L234-16S | 16-07-2002 | Ross | 009 | 120 | 50 | 50 | 165 K | 158 K | 0205 | 0206 | 0207 |  |
| L240-13S | 16-07-2002 | Andrew | 007 | 30 | 50 | 50 | 220 K | 300 K | 0209 | 0210 | 0211 |  |
| L240-10S | 16-07-2002 | Andrew | 007 | 30 | 50 | 50 | 190 K | 80 K | 0209 | 0210 | 0211 |  |
| L240-07S | 16-07-2002 | Andrew | 007 | 30 | 50 | 50 | 57 K | 167 K | 0209 | 0210 | 0211 | No Hx |
| L240-04S | 16-07-2002 | Andrew | 007 | 30 | 50 | 50 | 140 K | 50 K | 0209 | 0210 | 0211 |  |
| L240-01S | 16-07-2002 | Andrew | 007 | 30 | 50 | 50 | 107 K | 91 K | 0209 | 0210 | 0211 |  |
| L240-02N | 16-07-2002 | Andrew | 007 | 30 | 50 | 50 | 90 K | 101 K | 0209 | 0210 | 0211 |  |
| L240-05N | 16-07-2002 | Andrew | 007 | 30 | 50 | 50 | 168 K | 186 K | 0209 | 0210 | 0211 |  |
| L240-08N | 16-07-2002 | Andrew | 007 | 30 | 50 | 50 | 83 K | 50 K | 0209 | 0210 | 0211 |  |
| L240-11N | 16-07-2002 | Andrew | 007 | 30 | 50 | 50 | 170 K | 100 K | 0209 | 0210 | 0211 |  |
| L240-14N | 16-07-2002 | Andrew | 007 | 30 | 50 | 50 | 42 K | 24 K | 0209 | 0210 | 0211 |  |
| L234-04S | 17-07-2002 | Ross | 009 | 120 | 50 | 50 | 19 K | 48 K | 0205 | 0206 | 0207 |  |
| L234-07S | 17-07-2002 | Ross | 009 | 120 | 50 | 50 | 37 K | 40 K | 0205 | 0206 | 0207 |  |
| L234-10S | 17-07-2002 | Ross | 009 | 120 | 50 | 50 | 130 K | 110 K | 0205 | 0206 | 0207 |  |
| L234-13S | 17-07-2002 | Ross | 009 | 120 | 50 | 50 | 79 K | 85 K | 0205 | 0206 | 0207 |  |
| L234-16S | 17-07-2002 | Ross | 009 | 120 | 50 | 50 | 90 K | 45 K | 0205 | 0206 | 0207 |  |
| L234-19S | 17-07-2002 | Ross | 009 | 120 | 50 | 50 | 24 K | 36 K | 0205 | 0206 | 0207 |  |
| L234-22S | 17-07-2002 | Ross | 009 | 120 | 50 | 50 | 80 K | 140 K | 0205 | 0206 | 0207 |  |
| L234-01S | 17-07-2002 | Ross | 009 | 120 | 50 | 50 | 37 K | 24 K | 0205 | 0206 | 0207 |  |
| L234-02N | 17-07-2002 | Ross | 009 | 120 | 50 | 50 | 40 K | 54 K | 0205 | 0206 | 0207 |  |
| l240-20s | 17-07-2002 | Andrew | 007 | 30 | 50 | 50 | 34 K | 26 K | 0209 | 0210 | 0211 |  |
| l240-17s | 17-07-2002 | Andrew | 007 | 30 | 50 | 50 | 52 K | 66 K | 0209 | 0210 | 0211 |  |
| l248-14s | 17-07-2002 | Andrew | 007 | 30 | 50 | 50 | 44 K | 66 K | 0209 | 0210 | 0211 |  |
| l248-17s | 17-07-2002 | Andrew | 007 | 30 | 50 | 50 | 93 K | 88 K | 0209 | 0210 | 0211 |  |
| l248-20s | 17-07-2002 | Andrew | 007 | 30 | 50 | 50 | 58 K | 53 K | 0209 | 0210 | 0211 |  |
| l254-20s | 17-07-2002 | Andrew | 007 | 30 | 50 | 50 | 52 K | 40 K | 0209 | 0210 | 0211 |  |
| l254-17s | 17-07-2002 | Andrew | 007 | 30 | 50 | 50 | 11 K | 53 K | 0209 | 0210 | 0211 |  |
| l254-14s | 17-07-2002 | Andrew | 007 | 30 | 50 | 50 | 18 K | 50 K | 0209 | 0210 | 0211 |  |
| l254-11s | 17-07-2002 | Andrew | 007 | 30 | 50 | 50 | 90 K | 19 K | 0209 | 0210 | 0211 |  |
| L224-05N | 18-07-2002 | Ross | 009 | 120 | 50 | 50 | 15 K | 24 K | 0205 | 0206 | 0207 |  |
| L224-08N | 18-07-2002 | Ross | 009 | 120 | 50 | 50 | 50 K | 50 K | 0205 | 0206 | 0207 |  |
| L224-10N | 18-07-2002 | Ross | 009 | 120 | 50 | 46 | 20 K | 40 K | 0205 | 0206 | 0207 |  |
| L240-10S | 18-07-2002 | Ross | 009 | 120 | 50 | 50 | 50 K | 90 K | 0205 | 0206 | 0207 |  |
| L248-10S | 18-07-2002 | Ross | 009 | 120 | 50 | 50 | 150 K | 150 K | 0205 | 0206 | 0207 |  |
| L248-07S | 18-07-2002 | Ross | 009 | 120 | 50 | 50 | 90 K | 100 K | 0205 | 0206 | 0207 |  |
| L240-04S | 18-07-2002 | Ross | 009 | 120 | 50 | 50 | 50 K | 125 K | 0205 | 0206 | 0207 |  |
| L248-01S | 18-07-2002 | Ross | 009 | 120 | 50 | 50 | 50 K | 75 K | 0205 | 0206 | 0207 |  |
| L248-02N | 18-07-2002 | Ross | 009 | 120 | 50 | 50 | 50 K | 50 K | 0205 | 0206 | 0207 |  |
| L254-07S | 18-07-2002 | Ross | 009 | 120 | 50 | 50 | 50 K | 45 K | 0205 | 0206 | 0207 |  |
| L254-10S | 18-07-2002 | Ross | 009 | 120 | 50 | 50 | 105 K | 100 K | 0205 | 0206 | 0207 |  |
| L254-03S | 18-07-2002 | Andrew | 007 | 30 | 50 | 50 | 1.8K | 72 K | 0209 | 0210 | 0211 |  |
| L254-00 | 18-07-2002 | Andrew | 007 | 30 | 50 | 50 | 19 K | 16 K | 0209 | 0210 | 0211 |  |
| L254-03N | 18-07-2002 | Andrew | 007 | 30 | 50 | 50 | 39 K | 42 K | 0209 | 0210 | 0211 | Hx & Hz swapped |
| L254-06N | 18-07-2002 | Andrew | 007 | 30 | 50 | 50 | 35 K | 19 K | 0209 | 0210 | 0211 |  |
| L254-09N | 18-07-2002 | Andrew | 007 | 30 | 50 | 50 | 40 K | 41 K | 0209 | 0210 | 0211 |  |
| L254-12N | 18-07-2002 | Andrew | 007 | 30 | 50 | 50 | 39 K | 32 K | 0209 | 0210 | 0211 |  |
| L254-14N | 18-07-2002 | Andrew | 007 | 30 | 50 | 50 | 20 K | 24 K | 0209 | 0211 | 0210 |  |
| L248-14N | 18-07-2002 | Andrew | 007 | 30 | 50 | 50 | 32 K | 64 K | 0209 | 0211 | 0210 |  |
| L248-11N | 18-07-2002 | Andrew | 007 | 30 | 50 | 50 | 35 K | 45 K | 0209 | 0211 | 0210 |  |
| L248-08N | 18-07-2002 | Andrew | 007 | 30 | 50 | 50 | 100 K | 78 K | 0209 | 0211 | 0210 |  |
| L288-14N | 19-07-2002 | Ross | 009 | 120 | 50 | 50 | 24 K | 44 K | 0205 | 0206 | 0207 |  |
| L288-11N | 19-07-2002 | Ross | 009 | 120 | 50 | 50 | 40 K | 30 K | 0205 | 0206 | 0207 |  |
| L288-08N | 19-07-2002 | Ross | 009 | 120 | 50 | 50 | 40 K | 50 K | 0205 | 0206 | 0207 |  |
| L288-05N | 19-07-2002 | Ross | 009 | 120 | 50 | 50 | 50 K | 40 K | 0205 | 0206 | 0207 |  |
| L288-02N | 19-07-2002 | Ross | 009 | 120 | 50 | 50 | 15 K | 15 K | 0205 | 0206 | 0207 |  |
| L288-01S | 19-07-2002 | Ross | 009 | 120 | 50 | 50 | 10 K | 10 K | 0205 | 0206 | 0207 |  |
| L288-04S | 19-07-2002 | Ross | 009 | 120 | 50 | 50 | 65 K | 80 K | 0205 | 0206 | 0207 |  |
| L288-07S | 19-07-2002 | Ross | 009 | 120 | 50 | 50 | 110 K | 120 K | 0205 | 0206 | 0207 |  |
| L288-10S | 19-07-2002 | Ross | 009 | 120 | 50 | 50 | 75 K | 90 K | 0205 | 0206 | 0207 |  |
| L288-13S | 19-07-2002 | Ross | 009 | 120 | 50 | 50 | 110 K | 120 K | 0205 | 0206 | 0207 |  |
| L288-16S | 19-07-2002 | Ross | 009 | 120 | 50 | 50 | 125 K | 130 K | 0205 | 0206 | 0207 |  |
| L288-19S | 19-07-2002 | Ross | 009 | 120 | 50 | 50 | 40 K | 30 K | 0205 | 0206 | 0207 |  |
| L248-05N | 19-07-2002 | Andrew | 007 | 30 | 50 | 50 | 23 K | 30 K | 0209 | 0104 | 0211 | shift 0.04883 ms |
| L271-05N | 19-07-2002 | Andrew | 007 | 30 | 50 | 50 | 43 K | 41 K | 0209 | 0104 | 0211 |  |
| L271-08N | 19-07-2002 | Andrew | 007 | 30 | 50 | 50 | 64 K | 34 K | 0209 | 0104 | 0211 |  |
| L271-11N | 19-07-2002 | Andrew | 007 | 30 | 50 | 50 | 29 K | 20 K | 0209 | 0104 | 0211 | problem with Hy |
| L271-14N | 19-07-2002 | Andrew | 007 | 30 | 50 | 50 | 28 K | 16 K | 0209 | 0104 | 0211 | problem with Hy |
| L265-14N | 19-07-2002 | Andrew | 007 | 30 | 50 | 50 | 28 K | 32 K | 0209 | 0104 | 0211 | problem with Hy |
| L265-11N | 19-07-2002 | Andrew | 007 | 30 | 50 | 50 | 72 K | 63 K | 0209 | 0104 | 0211 | problem with Hy |
| L265-08N | 19-07-2002 | Andrew | 007 | 30 | 50 | 50 | 107 K | 44 K | 0209 | 0104 | 0211 | problem with Hy |
| L265-05N | 19-07-2002 | Andrew | 007 | 30 | 50 | 50 | 30 K | 45 K | 0209 | 0104 | 0211 | problem with Hy |
| L296-30N | 20-07-2002 | Ross | 009 | 120 | 50 | 50 | 20 K | 25 K | 0205 | 0206 | 0207 | Hy backwards |
| L296-27N | 20-07-2002 | Ross | 009 | 120 | 50 | 50 | 15 K | 19 K | 0205 | 0206 | 0207 |  |
| L296-24N | 20-07-2002 | Ross | 009 | 120 | 50 | 50 | 30 K | 50 K | 0205 | 0206 | 0207 |  |
| L296-21N | 20-07-2002 | Ross | 009 | 120 | 50 | 50 | 30 K | 25 K | 0205 | 0206 | 0207 |  |
| L296-18N | 20-07-2002 | Ross | 009 | 120 | 50 | 50 | 50 K | 70 K | 0205 | 0206 | 0207 |  |
| L296-15N | 20-07-2002 | Ross | 009 | 120 | 50 | 50 | 30 K | 40 K | 0205 | 0206 | 0207 |  |
| L296-12N | 20-07-2002 | Ross | 009 | 120 | 50 | 50 | 45 K | 60 K | 0205 | 0206 | 0207 |  |
| L296-09N | 20-07-2002 | Ross | 009 | 120 | 50 | 50 | 25 K | 45 K | 0205 | 0206 | 0207 |  |
| L296-06N | 20-07-2002 | Ross | 009 | 120 | 50 | 50 | 30 K | 40 K | 0205 | 0206 | 0207 |  |
| L266-03N | 20-07-2002 | Andrew | 007 | 30 | 50 | 50 | 28 K | 34 K | 0209 | 0208 | 0211 |  |
| L266-00N | 20-07-2002 | Andrew | 007 | 30 | 50 | 50 | 31 K | 70 K | 0209 | 0208 | 0211 |  |
| L266-03S | 20-07-2002 | Andrew | 007 | 30 | 50 | 50 | 21 K | 59 K | 0209 | 0208 | 0211 |  |
| L266-06S | 20-07-2002 | Andrew | 007 | 30 | 50 | 50 | 0.8 K | 1 K | 0209 | 0208 | 0211 |  |
| L266-09S | 20-07-2002 | Andrew | 007 | 30 | 50 | 50 | 48 K | 27 K | 0209 | 0208 | 0211 |  |
| L266-12S | 20-07-2002 | Andrew | 007 | 30 | 50 | 50 | 32 K | 66 K | 0209 | 0208 | 0211 |  |
| L266-15S | 20-07-2002 | Andrew | 007 | 30 | 50 | 50 | 2 K | 3 K | 0209 | 0208 | 0211 |  |
| L266-18S | 20-07-2002 | Andrew | 007 | 30 | 50 | 50 | 61 K | 52 K | 0209 | 0208 | 0211 | Ex bad |
| L266-20S | 20-07-2002 | Andrew | 007 | 30 | 50 | 50 | 27 K | 32 K | 0209 | 0208 | 0211 |  |
| L271-20S | 20-07-2002 | Andrew | 007 | 30 | 50 | 50 | 42 K | 23 K | 0209 | 0208 | 0211 |  |
| L266-06N | 21-07-2002 | Andrew | 007 | 30 | 50 | 50 | 28 K | 28 K | 0209 | 0208 | 0211 | shift -0.09776 ms |
| L266-09N | 21-07-2002 | Andrew | 007 | 30 | 50 | 50 | 25 K | 35 K | 0209 | 0208 | 0211 | shift -0.1953 ms |
| L266-12N | 21-07-2002 | Andrew | 007 | 30 | 50 | 50 | 45 K | 35 K | 0209 | 0208 | 0211 | shift -0.3174 ms |
| L266-14N | 21-07-2002 | Andrew | 007 | 30 | 50 | 50 | 25 K | 25 K | 0209 | 0208 | 0211 | shift -0.4883 ms |
| L271-14N | 21-07-2002 | Andrew | 007 | 30 | 50 | 50 | 20 K | 11 K | 0209 | 0208 | 0211 | shift -0.6592 ms |
| L271-11N | 21-07-2002 | Andrew | 007 | 30 | 50 | 50 | 25 K | 15 K | 0209 | 0208 | 0211 | shift -0.8789 ms |
| L271-01N | 21-07-2002 | Andrew | 007 | 30 | 50 | 50 | 20 K | 24 K | 0209 | 0208 | 0211 | shift -1.270 ms |
| L266-06N | 21-07-2002 | Andrew | 007 | 30 | 50 | 50 | 1 K | 2 K | 0209 | 0208 | 0211 | shift -1.465 ms |
| L304-08S | 21-07-2002 | Ross | 009 | 120 | 50 | 50 | 60 K | 70 K | 0205 | 0206 | 0207 |  |
| L304-05S | 21-07-2002 | Ross | 009 | 120 | 50 | 50 | 80 K | 90 K | 0205 | 0206 | 0207 | shift -0.02441 ms |
| L304-02S | 21-07-2002 | Ross | 009 | 120 | 50 | 50 | 70 K | 80 K | 0205 | 0206 | 0207 | shift -0.04883 ms |
| L304-01N | 21-07-2002 | Ross | 009 | 120 | 50 | 50 | 30 K | 40 K | 0205 | 0206 | 0207 | shift -0.02441 ms |
| L304-04N | 21-07-2002 | Ross | 009 | 120 | 50 | 50 | 30 K | 40 K | 0205 | 0206 | 0207 | shift -0.09776 ms |
| L304-07N | 21-07-2002 | Ross | 009 | 120 | 50 | 50 | 40 K | 50 K | 0205 | 0206 | 0207 | shift -0.2441 ms |
| L304-10N | 21-07-2002 | Ross | 009 | 120 | 50 | 50 | 90 K | 80 K | 0205 | 0206 | 0207 | shift -0.3418 ms |
| L304-13N | 21-07-2002 | Ross | 009 | 120 | 50 | 50 | 40 K | 70 K | 0205 | 0206 | 0207 | shift -0.8057 ms |
| L304-16N | 21-07-2002 | Ross | 009 | 120 | 50 | 50 | 35 K | 40 K | 0205 | 0206 | 0207 | shift -0.6348 ms |
| L304-19N | 21-07-2002 | Ross | 009 | 120 | 50 | 50 | 50 K | 40 K | 0205 | 0206 | 0207 | shift -0.8057 ms |
| L304-22N | 21-07-2002 | Ross | 009 | 120 | 50 | 50 | 20 K | 25 K | 0205 | 0206 | 0207 | shift -1.001 ms |
| L304-26N | 21-07-2002 | Ross | 009 | 120 | 50 | 50 | 40 K | 50 K | 0205 | 0206 | 0207 | shift -1.465 ms |
| L304-30N | 21-07-2002 | Ross | 009 | 120 | 50 | 50 | 90 K | 100 K | 0205 | 0206 | 0207 | shift -1.636 ms |
| L304-20S | 22-07-2002 | Ross | 009 | 120 | 50 | 50 | 40 K | 30 K | 0205 | 0206 | 0207 |  |
| L304-17S | 22-07-2002 | Ross | 009 | 120 | 50 | 50 | 80 K | 70 K | 0205 | 0206 | 0207 | shift 0.02441 ms |
| L304-14S | 22-07-2002 | Ross | 009 | 120 | 50 | 50 | 70 K | 80 K | 0205 | 0206 | 0207 | shift 0.04883 ms |
| L304-11S | 22-07-2002 | Ross | 009 | 120 | 50 | 50 | 40 K | 30 K | 0205 | 0206 | 0207 | shift 0.07324 ms |
| L276-16S | 22-07-2002 | Ross | 009 | 120 | 50 | 50 | 20 K | 30 K | 0205 | 0206 | 0207 | shift 0.04883 ms |
| L276-13S | 22-07-2002 | Ross | 009 | 120 | 50 | 50 | 30 K | 40 K | 0205 | 0206 | 0207 |  |
| L276-10S | 22-07-2002 | Ross | 009 | 120 | 50 | 50 | 30 K | 25 K | 0205 | 0206 | 0207 | shift 0.04883 ms |
| L276-07S | 22-07-2002 | Ross | 009 | 0 | 50 | 50 | 30 K | 40 K | 0205 | 0206 | 0207 | saturated Hy |
| L276-01S | 22-07-2002 | Ross | 009 | 120 | 50 | 50 | 25 K | 30 K | 0205 | 0206 | 0207 | shift 0.2197 ms |
| L276-02N | 22-07-2002 | Ross | 009 | 120 | 50 | 50 | 30 K | 30 K | 0205 | 0206 | 0207 | shift 0.2686 ms |
| L271-17S | 22-07-2002 | Andrew | 007 | 30 | 50 | 50 | 13 k | 9 K | 0209 | 0208 | 0211 |  |
| L271-14S | 22-07-2002 | Andrew | 007 | 30 | 50 | 50 | 13 k | 9 K | 0209 | 0208 | 0211 |  |
| L271-14S | 22-07-2002 | Andrew | 007 | 30 | 50 | 50 | 45 K | 36 K | 0209 | 0208 | 0211 |  |
| L271-11S | 22-07-2002 | Andrew | 007 | 30 | 50 | 50 | 42 K | 47 K | 0209 | 0208 | 0211 |  |
| L271-08S | 22-07-2002 | Andrew | 007 | 30 | 50 | 50 | 45 K | 50 K | 0209 | 0208 | 0211 | SWAP XZ |
| L271-05S | 22-07-2002 | Andrew | 007 | 30 | 50 | 50 | 35 K | 35 K | 0209 | 0208 | 0211 |  |
| L276-20N | 22-07-2002 | Andrew | 007 | 30 | 50 | 50 | 30 K | 44 K | 0209 | 0208 | 0211 |  |
| L276-17N | 22-07-2002 | Andrew | 007 | 30 | 50 | 50 | 28 K | 27 K | 0209 | 0208 | 0211 |  |
| L276-14N | 22-07-2002 | Andrew | 007 | 30 | 50 | 50 | 27 K | 31 K | 0209 | 0208 | 0211 |  |
| L276-11N | 22-07-2002 | Andrew | 007 | 30 | 50 | 50 | 46 K | 44 K | 0209 | 0208 | 0211 |  |
| L276-08N | 22-07-2002 | Andrew | 007 | 30 | 50 | 50 | 55 K | 45 K | 0209 | 0208 | 0211 |  |
| L276-05N | 22-07-2002 | Andrew | 007 | 30 | 50 | 50 | 44 K | 26 K | 0209 | 0208 | 0211 |  |

  - Data formats

Original field data were archived in time-series files in a mixed binary format as written by the Metronix acquisition program GMS183b. All times series data are stored in a directory corresponding to the site name within a directory designated by the date of acquisition. Individual files within the directory follow the convention.

ssscrrtb.ATS

where:

| sss | = ADU06 serial number |
| --- | --- |
| c | = ADU06 channel number (a=1..h=8) |
| rr | = Run number |
| t | = channel type (a=Ex, b=Ey, x=Hx, y=Hy, z=Hz) |
| b | = Sample rate (a=40960, b=4096, c=64, f=2048 or 512 Hz) |

  - EDI files:

XXYYn.EDI, defines an EDI file obtained by robust processing.

Where XX = grid easting in units of 100m

and YY = grid northing in units of 100m

Data were normally archived in the following formats:

SEG-Electromagnetic Data Interchange (EDI) format containing all MT parameters;

Binary files in Metronix GMS183b format containing acquisition information and time-series.

The SEG MT/EMAP Data Interchange Standard is described in Wight, 1988. Since this is an extremely lengthy standard (91 pages), the reader is referred to that document for details.

- METRONIX TIME SERIES: ATS FORMAT

| Byte off-set | ADR | Byte | Type | Name | Info |
| --- | --- | --- | --- | --- | --- |
| 1 | 000H | 2 | INTEGER | Header Length | Length of Header in Byte |
| 3 | 002H | 2 | INTEGER | Header Version | Version Number of Header (*100)4 |
| 5 | 004H | 4 | LONGINT | Samples | Number of Samples |
| 9 | 008H | 4 | SINGLE | Sample Freq. | Sample Frequency |
| 13 | 00CH | 4 | LONGINT |  | Start Time of Measurement (in seconds since 1.1.70) |
| 17 | 010H | 8 | DOUBLE | LSBV | LSB Unit in mV2 |
| 25 | 018H | 8 | BYTE | Reserved | Reserved |
| 33 | 020H | 2 | INTEGER |  | Serial number of ADU06 |
| 35 | 022H | 2 | INTEGER |  | Serial number of ADB06 (ADC Board) |
| 37 | 024H | 1 | BYTE |  | Channel number (0...7) |
| 38 | 025h | 1 | BYTE | Reserved | Reserved |
| 39 | 026h | 2 | CHAR |  | Channel Type (Ex,Ey,Hx,Hy,Hz) |
| 41 | 028H | 6 | CHAR |  | Sensor Type (MFS05, BF4...) |
| 47 | 02EH | 2 | INTEGER |  | Serial number of sensor |
| 49 | 030H | 4 | SINGLE |  | x1 coordinates of Dipole(m) |
| 53 | 034H | 4 | SINGLE |  | y1 |
| 57 | 038H | 4 | SINGLE |  | z2 |
| 61 | 03CH | 4 | SINGLE |  | x2 coordinates of Dipole (m) |
| 65 | 040H | 4 | SINGLE |  | y2 |
| 69 | 044H | 4 | SINGLE |  | z2 |
| 73 | 048H | 4 | SINGLE |  | e-field dipole length (m) |
| 77 | 04CH | 4 | SINGLE |  | Angle (0°=north) (degrees °) |
| 81 | 050H | 4 | SINGLE |  | Probe resistivity (ohm) |
| 85 | 054H | 4 | SINGLE |  | DC offset voltage3 (mV) |
| 89 | 058H | 4 | SINGLE |  | Internal gain amplification (1 or 30)4 |
| 93 | 05CH | 4 | BYTE |  | Reserved |
| 97 | 060H | 4 | LONG |  | Latitude (msec) |
| 101 | 064H | 4 | LONG |  | Longitude (msec) |
| 105 | 068H | 4 | LONG |  | Elevation (cm) |
| 109 | 06CH | 1 | CHAR |  | Lat/Long Type: ‘U’ user defined, ‘G’ internal GPS clock |
| 110 | 06DH | 1 | CHAR |  | Type of additional coordinates: ’U’ UTM ‘G’ Gauss-Kruger |
| 111 | 06EH | 2 | INTEGER |  | Reference median |
| 113 | 070H | 8 | DOUBLE |  | X coordinate |
| 121 | 078H | 8 | DOUBLE |  | Y coordinate |
| 129 | 080H | 1 | CHAR |  | GPS/CLK status: ‘G’ GPS locked, ‘C’ CLK synchronized, ‘N’ CLK not synchronized |
| 130 | 089H | 1 | BYTE |  | Approximate accuracy of GPS/CLK: 9 means accuracy of 109 |
| 131 | 082H | 14 | BYTE |  | Reserved |
| 145 | 090H | 12 | CHAR |  | Survey header file name |
| 157 | 09CH | 4 | CHAR |  | Type of measurement: MT or CSAMT |
| 161 | 0A0H | 12 | CHAR |  | Log file of system self test |
| 173 | 0ACH | 2 | CHAR |  | Result of self test ‘OK’ or ‘NO’ |
| 175 | 0AEH | 2 | BYTE |  | Reserved |
| 177 | 0B0H | 2 | INTEGER |  | Number of calibration frequencies in file |
| 179 | 0B2H | 2 | INTEGER |  | Length of frequency entry (32 byte) |
| 181 | 0B4H | 2 | INTEGER |  | Version of calibration format (*100) |
| 183 | 0B6H | 2 | INTEGER |  | Start address of calibration information in header (400H) |
| 185 | 0B8H | 8 | BYTE |  | Reserved |
| 193 | 0C0H | 12 | CHAR |  | File name of ADU06 cal file |
| 205 | 0CCH | 4 | LONG |  | Date/time of calibration |
| 209 | 0D0H | 12 | CHAR |  | File name of sensor calibration |
| 221 | 0DCH | 4 | LONG |  | Date/time of calibration |
| 225 | 0E0H | 4 | SINGLE |  | Power-line freq. 1 |
| 229 | 0E4H | 4 | SINGLE |  | Power-line freq.2 |
| 233 | 0E8H | 8 | BYTE |  | Reserved |
| 241 | 0F0H | 4 | SINGLE |  | CSAMT transmitter frequency |
| 245 | 0F4H | 2 | INTEGER |  | CSAMT time series block |
| 247 | 0F6H | 2 | INTEGER |  | CSAMT stacks/block |
| 249 | 0F8H | 4 | LONG |  | CSAMT block length |
| 253 | 0FCH | 4 | BYTE |  | Reserved |
| 257 | 100H | 16 | CHAR |  | Client |
| 273 | 110H | 16 | CHAR |  | Contractor |
| 289 | 120H | 16 | CHAR |  | Area |
| 305 | 130H | 16 | CHAR |  | Survey ID |
| 321 | 140H | 16 | CHAR |  | Operator |
| 337 | 150h | 112 | CHAR |  | Reserved |
| 449 | 1C0H | 64 | CHAR |  | Weather |
| 513 | 200H | 512 | CHAR |  | Comments |
| 1025 | 400H | 32 |  |  | Calibration frequency 1 |
| 1025 | 400H | 4 | SINGLE |  | Frequency |
| 1029 | 404H | 4 | SINGLE |  | Amplitude: e-field (V) - h-field (nT/V) |
| 1033 | 408H | 4 | SINGLE |  | Phase (degrees) |
| 1037 | 40ch | 4 | SINGLE |  | Accuracy of amplitude (%) |
| 1041 | 410H | 4 | SINGLE |  | Accuracy of phase (+/- degrees) |
| 1045 | 414H | 12 | BYTE |  | Reserved |
| 1057 | 420H | 32 |  |  | Calibration frequency 2 |
|  | 400H+(n.1)*20H | 32 |  |  | Calibration frequency n |
|  | 400H+n*20H | 4 Samples | LONGINT |  | Time series data |

- AMT PARAMETERS

The audiomagnetotelluric method is a means of determining the resistivity distribution of the earth through the measurement of time varying electric and magnetic fields at the surface. At each AMT site, data from five channels are recorded as a function of time, which are referred to as time series. These channels are indicated in the following figure, and correspond to three orthogonal magnetic field components (designated Hx, Hy, and Hz), and two horizontal electric field components (designated Ex and Ey). Note that a right-hand coordinate system is used and z is positive downwards.

Figure 1: Coordinate axes and component identifications for 5-component AMT site.

As an electromagnetic method, magnetotellurics depends on Maxwell’s law stating that a time-varying magnetic field induces an electric field in a conductor. The source fields are the time-varying horizontal magnetic fields (Hx and Hy), which are generated by two distinct phenomena. The high frequency source fields, greater than 10 Hz, are generated by lightning discharges of distant electrical storms. The low frequency source fields are generated by the interaction of charged particles, solar wind, with the earth’s ionosphere. The output of the source fields convolved with the earth consists of the horizontal electric field (Ex, and Ey) and the vertical magnetic field (Hz). Thus, ideally, the electrical nature of the earth (i.e. the impedance) can be determined through the transfer function of the measured input and output signals:

  - Measured quantities

The actual parameters measured in the field are the time-varying voltage outputs of the electric and magnetic field sensors: Ex, Ey, Hx, Hy, and Hz. Data were recorded in 2 bands, 40960 and 2048 Hz sample rates.

  - Computed functions

The measured parameters, the electric and magnetic field values, are transformed into the frequency domain using FFT procedures, and convolved with the sensor responses to give the complex values of electric and magnetic fields at specific frequencies.

The resulting Fourier-transformed spectral estimates are combined into a spectral crosspower matrix relating all of the measured electromagnetic fields at discrete frequency values. If the spectral values of two channels at frequency fj in the channel bounded by frequencies between fj-m and fj+m are A and B (complex numbers), then

A(fj),A(fj)=12m+1k=j−mj+mAkAk=AkAk

define the autopower AjAj ; and

A(fj),B(fj)=12m+1k=j−mj+mAkBk=AkBk

define the crosspower  ,where the * indicates the complex conjugate.

- The impedance tensor is calculated directly from the crosspower matrix, via relationships of the form

The relationship between the five measured components at each site are contained in the impedance tensor (Zij) and the tipper transfer function (Ti), expressed by:

Ex = ZxyHy + ZxxHx

Ey = ZyxHx + ZyyHy

Hz = TxHx + TyHy

The impedance tensor and the crosspower matrix are used to derive more practical parameters for interpretation and data quality assessment.

    - Data interpretation parameters

The interpretation parameters are calculated using the standard definitions of Vozoff (1991), and are described here in simplified form.

Apparent Resistivity - scaled magnitude of the ratio of each orthogonal E and H pair, with associated variances, i.e.

ρij=15fZij2

Impedance Phase - impedance phase of each orthogonal E and H pair, with associated variances.

Impedance Rotation - present rotation direction of Zxy (i.e. it can be a fixed, user-specified rotation angle, or that defined as impedance strike).

Impedance Strike - angle which minimizes |Zxx()|2 + |Zyy()|2. In an ideal 2-D environment, one component will be parallel to strike (transverse electric, or TE mode), and the other will be perpendicular to strike (transverse magnetic, or TM mode).

Tipper Strike - direction which maximizes the cross power of horizontal and vertical magnetic field components + 90 degrees.

Tipper Magnitude – magnitude of the vertical magnetic field with respect to the total horizontal magnetic field.

Tipper = SQRT(|Tx|2 + |Ty|2)

Impedance Skew - impedance tensor ratio, 3-D indicator, invariant with rotation.

|Zxx+Zyy|/|Zxy-Zyx|

Impedance Ellipticity - impedance tensor ratio, 3-D indicator, dependent upon rotation.

|Zxx()-Zyy()|/|Zxy()+Zyx()|

- PHASE MAPS