Line: DB-5

# GEOMETRY

# Used crooked line geometry algorithm

# VELOCITY

Used velocity function from Jonathan’s sonic log for hole 140-09, near station 3501 on DB-3.  Picked a brute velocity function to be used for NMO correction of:

Time: 137ms		Velocity: 1900 m/s

Time: 148ms		Velocity: 2800 m/s

Time: 163ms		Velocity: 3300 m/s

# FILTER TESTS

Frequency content was tested over the following bandwidths:

No Filter

8/12 – 30/40 Hz

30/40 – 70/80 Hz

50/60 – 120/140 Hz

60/70 – 150/170 Hz

80/90 – 160/180 Hz

100/120 – 180/200 Hz

120/140 – 220/240 Hz

140/160 – 280/300 Hz

Chose filter of 50/60 – 200/240 Hz for now, to stack frequencies above ground roll, which hasn’t been removed yet.

# SPECTRAL ANALYSIS

Spectral analysis on the raw shot records revealed a maximum usable frequency of near 300 Hz

# MUTE

A stretch mute was applied during NMO correction to mute NMO stretching above 80%.

# DECONVOLUTION

A simple zero-phase spiking decon was used before stack to increase temporal resolution.  The following parameters were applied:

Decon operator length: 120 ms

Operator white noise level (pre-whitening): 1.0%

Decon window: 50:186:500-2600/ 50:497:1000-2800

That is: FFID 50; Channel 186 (8.50 m offset); Time 500ms to 2600ms

Channel 497 (920.58 m offset); Time 1000ms to 2800ms

# TRACE REVERSALS

The raw shot records contained 10 reversed channels, 135-144 inclusive.  These were subsequently converted to normal polarity before further processing.

## FIRST BREAKS

First breaks were picked manually using Trace Display’s first break editor.  Due to near offset noise, delineating the first breaks was difficult, but a bandpass filter of 70/80 – 200/240Hz was used to remove low frequency reverberations.  The first break velocity increases at roughly 140ms two-way time, suggesting a second layer.  First breaks were picked after scaling the data to 10% of default gain.

# GAIN

Automatic Gain Control was applied twice during the brute stack run.  Once to the raw shot records, with a mean scalar and a 50ms window.  Then, post stack with a mean scalar and a 100ms window.

# REFRACTION ANALYSIS

Several refraction statics solutions were discussed and applied, but not agreed upon at this point.

## RESIDUAL STATICS

Feb 12-14th: Max Power statics was applied to the data with smash=5 cdps and window gate=400 ms at a horizon of 300 ms.