##### APPENDIX 5 - STAR 3D PROCESSING FLOW CHART

## SEG-D INPUT

## SORTING OF SHOTS

Ensemble Redefine: Mode of Application – absolute difference of headers

## CALCULATION OF GEOMETRY

Crooked line geometry algorithm

## APPLICATION OF GEOMETRY

Inline geometry header load: Primary header to match database – FFID

Secondary header to match database – NONE

Match by valid trace number – NO

Drop traces with null CDP header – NO

Drop traces with null receiver header – NO

Verbose Diagnostics – NO

## SPHERICAL DIVERGENCE CORRECTION

True Amplitude Recovery: Basis for spherical spreading – 1/tv2

Velocity treated as space variable – NO

Maximum application time – 2000 ms

## SCALING

Automatic Gain Control (AGC): Type of scalar – MEAN

Window size – 300 ms

Window position – CENTRED

Robust scaling – NO

## FREQUENCY FILTERING

Bandpass Filter: Type of filter – SINGLE FILTER

Type of filter specification – ORMSBY BANDPASS

Phase of filter – ZERO PHASE

Apply a notch filter – NO

Filter Frequency Values – 40/50 – 200/240 Hz

## DECONVOLUTION

Surface Consistent Decon:  Get decon gates from database – NO

Primary decon gate header  word – LIVE SOURCE NUMBER

Secondary decon gate header – ABSOLUTE VALUE OF OFFSET

Specify decon gate parameters – 1:0:0-100/1:100:0-300/1:300:0-500

#### SPECTRAL WHITENING

Time Variant Spectral Whitening:  Type of Filter – TIME-INVARIANT PARAMETERS

Spectral balancing scalar length – 250

Automatic filter panel design? – YES

Number of frequency panels – 10

Specify spectral balancing frequencies – 60, 70, 180, 190

Mode for amplitude restoration – LOG AVERAGE

#### STATICS

Apply Elevation Statics:  Final datum elevation – 420.0 m

Replacement velocity – 1500.0 m/s

Database math method – SURFACE SOURCE

NMO static method – ELEVATIONS

- LENGTH OF SMOOTHER: 51

Processing datum – NMO DATUM

Apply Residual Statics

#### VELOCITY ANALYSIS

Normal Moveout Correction: Direction for NMO application – FORWARD

Stretch mute percentage – 50%

Apply any remaining static during NMO? – YES

Get 3D dip velocities? – NO

Get velocities from database? – YES

Velocity parameter file – 3d rms edit

#### MUTING

Trace Muting: Re-apply previous mutes? – NO

Mute time reference – TIME 0

Type of Mute – TOP

Starting ramp – 10

Primary mute header word – LIVE SOURCE NUMBER

Secondary mute header word – ABSOLUTE VALUE OF OFFSET

Specify  mute parameters – 1:30:25/1:300:230/

Trace Muting: Re-apply previous mutes? – NO

Mute time reference – TIME 0

Type of Mute – BOTTOM

Starting ramp – 20

Primary mute header word – LIVE SOURCE NUMBER

Secondary mute header word – ABSOLUTE VALUE OF OFFSET

Specify  mute parameters – 1:30:150/1:100:400/1:120:2000/

#### FREQUENCY FILTERING

Bandpass Filter: Type of filter – SINGLE FILTER

Type of filter specification – ORMSBY BANDPASS

Phase of filter – MINIMUM PHASE

Apply a notch filter – NO

Filter Frequency Values – 60/70 – 180/200 Hz

#### SCALING

Automatic Gain Control (AGC): Type of scalar – MEAN

Window size – 200 ms

Window position – CENTRED

Robust scaling – NO

## STACKING

CDP/Ensemble Stack: Sort order for input ensembles – CDP

Method for trace summing – MEAN

Root power scalar for stack – 0.5

Normalization

Apply final datum statics after stack? – YES

## STACK PADDING

Pad 3D Stack Volume: Primary sort header word – ILINE_NO

First crossline header value – 1

Last crossline header value – 110

## MIGRATION

Explicit FD 3D Time Migration: Number of migration server nodes – 1

Local image copies? – NO

Memory limit per migration server – 1000MB

Velocity parameter file – 3D SIMPLE  INTVELS

FD migration algorithm – FULL 3D

Maximum dip to migrate – 30

Minimum in-line number – 2

Maximum in-line number – 98

Minimum x-line number – 1

Maximum x-line number – 110

Output sampling interval – 1ms

Maximum output time – 600ms

Minimum frequency to migrate – 70Hz

Maximum frequency to migrate – 180Hz

Percent to pad trace length – 30%

Top time taper – 50ms

Bottom time taper – 100ms

Re-apply trace mutes? – YES

Re-kill dead traces? – YES

## TIME TO DEPTH CORRECTION

Time to Depth Conversion: Conversion direction – TIME-TO-DEPTH

Maximum frequency of interest – 180Hz

Percent velocity scale factor – 100%

Get velocities from database – YES

Specify conversion velocity function –	 3D SIMPLE  INTVELS