Revised Feb 17, 2014

Read-Me.doc

By

R.f. Mereu

Professor Emeritus

Department of Earth Sciences

University of Western Ontario (UWO)

London, Ontario, Canada, N6A 5B7

The data set given on this disc was recorded during the GLIMPCE Great Lakes seismic experiment which was conducted in 1986. The data is the seismic refraction data recorded by various research groups using fixed stations. The sources were air-gun sources fired from a moving ship. More details are summarized as follows

Approximate

Line   Airgun spacing      Location            No Stations Recording Institution

(meter)        Lake Superior

A         333             N-S line              7  GSC north end line A

5  USGS – OBS along line A

6  Wisconsin South end line A

2  UWO  Michipicoten Is.)--Fan

North East Lake Superior

B         65              S-N line              1  UWO (Michipicoten Is)

S end Line B

South-East Lake Superior

F         65              N-S line              1  UWO (Michipicoten Is)

N end Line F

(Georgian Bay and Lake Huron)

J         65              E-W Line              4  UWO --- E Georgian Bay

1  UWO --- E-W On Bruce Peninsula

We have no refraction data for lines C, H and I

When I received the data from other institutions in 1986, it was on magnetic tapes in SEGY format. I converted that data into UWO format before our analysis and saved it on large main frame computer tapes. In the 1990s as these tapes began to deteriorate, I transferred the data to Exabyte tapes. When the CD writers came in the market, I copied the all my data sets to CD's in unformatted UWO mode. When I retired in 1996, I had to move to a smaller work space and had get rid of the old tapes. I did keep all the CD data tapes at home.

Following the recent request for GLIMPCE data which I received from Anne Trehu  (Oregon State University), I have rewritten all the GLIMPCE data into ASCII formatted UWO files. These are presented in this CD along with jpg plots of some of the record sections. I also present a file giving the data file header definitions along with instructions as to how the files may be read.

The following references show a list of our publications which give the  results of research conducted at the University of Western Ontario and research done in collaboration with others during the 1986-2000 period of time)

Note on the Band-pass Filter and Sampling Rates

The Glimpce data files had all been band-pass filtered to remove the effects of low frequency microseisms and high frequency noise.

The  band-pass filter limits and  sampling rates were used as  as follows:

Filter                                           Sampling Rate

Line A                4 – 30 Hz                                 125 for all GSC, USGS, and Wisconsin data sets

120 for UWO data.

Lines B and F     4 – 16 Hz                                 120

Line J                  4 – 25 Hz                                   120.

All filters used were non-realizable Butterworth filters of order 4. The parameters for these filters

are given in headers 31,32,33,34 and 35. The sampling rate parameter is given in header 9

Appropriate inverse filters may easily be designed to remove most of the effects of the band-pass filter if the user wishes to work with raw data.

References of Research on GLIMPCE Data

(Conducted at the University of Western Ontario and research done in collaboration with others during the 1986-2000 period of time)

Morel-a-l'Huissier, P., Trehu, A., Green, A.G., Hajnal, Z.,   Karl, J., Mereu, R., Meyer, R. P., Sexton, J., Hutchinson, D.,  and Shay, J. 1989.   GLIMPCE seismic experiments: long offset recordings.

The Glimpce seismic refraction working group. EOS, V 70, pp 841-853.

Epili D. and Mereu, R.F., 1989.

The Glimpce seismic experiment: Onshore refraction and wide-angle PmP observations from a fan profile over the  Lake Superior Midcontinent Rift System. In Proceedings of the  U7 Interdisciplinary Symposium of the IUGG, Vancouver 1987 on "Properties and Processes of the Earth's Lower Crust ".   Eds: R.F. Mereu, St. Mueller, and D. Fountain, American Geophysics Union Geophysical Monograph 51, IUGG Vol 6,  pp 93-101.

Green, A.G., Cannon,W.F., Milkereit, B., Hutchinson, D. R., Davidson,A., Behrendt,J., Spencer, C., Lee,M.W., Morel-a-L'Hussier,P., and Agna, W.F..1989. A “Glimpce” of the deep crust beneath the Great Lakes. In Proceedings of the  U7 Interdisciplinary Symposium of the IUGG, Vancouver

1987 on "Properties and Processes of the Earth's Lower Crust ".   Eds: R.F. Mereu, St. Mueller, and D. Fountain, American Geophysics Union Geophysical Monograph 51, IUGG Vol 6,  pp 93-101.

Behrendt, J., Green, A.G., Lee, M.W.,Hutchinson,D.R.,Cannon,W.F., Milkereit, B.,Agna,W.F., and Spencer, C. 1989, Custal extension in the Midcontinent Rift system-Results from Glimpce Deep Seismic Reflection profiles over Lakes Superior and Michigan. In Proceedings of the  U7 Interdisciplinary Symposium of the IUGG, Vancouver 1987 on "Properties and Processes of the Earth's Lower Crust ".   Eds: R.F. Mereu, St. Mueller, and D. Fountain, American Geophysics Union Geophysical Monograph 51, IUGG Vol 6,   pp 93-101.

Mereu, R.F., and Epili, D., 1990.  Pg Shingles: Preliminary results from the onshore GLIMPCE

refraction experiment. Tectonophysics. 173, pp 617-626.

Milkereit, B., Epili, D., Green, A.G., Mereu,R.f., and Morel-a-L'Hussier. 1990. Migration of wide-angle reflection data from the Grenville Front in Lake Huron. Journal of Geophysical Research V95, pp10987-10998.

Epili, D. 1990.  Crustal structure beneath the Great Lakes: Results from the GLIMPCE onshore

seismic data. Ph.D thesis, University of Western Ontario. (Supervised by R. Mereu)

Mereu, R.F., 1990.   The complexity of the crust from refraction/wide-angle   reflection data. In special issue on "Scattering and Attenuation of Seismic Waves". Eds. R.S. Wu and K. Aki.  PAGEOPH, V 132, PP 269-288.

Trehu, A., Morel-a-l'Huissier,.P., Meyer, R.P., Hajnal, Z., Karl, J.,  Mereu, R., Sexton, J., Shay, J., Chan, W.-K., Epili, D., Jefferson, T.,  Shih, X.R., Wendling, S., Milkereit, B., Green, A., and Hutchinson, D.,  1991. Imaging the Mid-continent Rift beneath Lake Superior using large aperture seismic data. Geophysical Research Letters, V18,  pp 625-628.

Epili, D, and Mereu, R.F., 1991.  The Grenville Front Tectonic Zone: Results from the 1986

GLIMPCE onshore seismic wide-angle reflection and refraction  experiment, Journal of Geophysical Research. V96, pp16,335-16,348.

Hamilton, D. 1991. Two-dimensional tomographical tweaking of the GLIMPCE data set across the Midcontinent Rift zone. M.Sc. Thesis. University of Western Ontario. (Supervised by R. Mereu)

Hamilton, D. and Mereu, R.F..1993. 2-D Tomographic imaging across the North American Mid-continent  Rift system. Geophys. J. Int. V112, pp 344-358.

Mereu, R.F. 2000. The effect of small random reflectors on the complexity of Pg and PmP coda. 2000.

Physics of the Earth and Planetary Interiors. V120, pp183-199.

Mereu, R.F. 2000.  The complexity of the crust and Moho under the southeastern Superior and Grenville Provinces of the Canadian Shield from  seismic wide-angle reflection data. Canadian Journal of Earth  Sciences. V 37: pp 439-458

Additional papers added by A. Trehu:

Trehu, A.M., B Milkereit, J. Shay, 1988, Structure of the lower crust beneath the Midcontinent Rift from migration of wide-angle reflections, EOS, Trans. Am. Geophys. Un. 69.

Shay, J., 1990, Structure of the crust beneath Lake Superior from forward modeling of large aperture data, M. Sc. Thesis, Oregon State University, 128 pp. (supervised by A. Trehu).

Shay, J. and Trehu, A.,  1993, Crustal structure of the midcontinent rift beneath Lake Superior, Tectonophysics, v. 225, 301-335.

Lutter, W., Trehu, A., and Nowack, R., 1993, Application of 2-D travel-time inversion to the midcontinent rift beneath Lake Superior,  Geophysical Research Letters, v. 20, 615-618.