- Observer's Daily Comments -

CLIENT: NORANDA INC.                         AREA: HALFMILE LAKE 3D                               DATE: 16/ SEPT/ 98

CONTRACTOR: WESTERN GEOPHYSICAL          PARTY NO: 779         JOB NO: 98-005       OBSERVERS: G. MITSCHKE / R. DAVIS

----------------------------------------------------------------------------------------------------------------------

WEATHER: PARTLY CLOUDY, MODERATE WINDS GUSTING TO 40 Km/h., 9 C. TO 15 C.

CAP SIM TEST; PELTON S/N-026, FILE 312.

CAP SIM TEST; PELTON S/N-038, FILE 313.

CAP SIM TEST; PELTON S/N-020, FILE 314.

RECEIVER LINES LAYED OUT TODAY:

R-3 - STATIONS 173 TO 132.				R-11 - STATIONS 173 TO 132.

R-5 - STATIONS 173 TO 132.				R-13 - STATIONS 173 TO 132.

R-7 - STATIONS 173 TO 132.				R-15 - STATIONS 173 TO 132.

R-9 - STATIONS 173 TO 132.

HELICOPTER PICKED UP (28) HELIBAGS OFF THE BACK OF THE SPREAD AND BROUGHT THEM THROUGH STAGING BEFORE BEING DEPLOYED

BACK ONTO LINES.

HELICOPTER PICKED UP (4) HELIBAGS WITH TRANSVERSE EQUIPMENT, THEN DEPLOYED (8) HELIBAGS FOR THE TRANSVERSE LINE.

EQUIPMENT DEPLOYED BY HELICOPTER:

R-3 - (7) HELIBAGS COVERING STATIONS 173 TO 131.	      R-11 - (7) HELIBAGS COVERING STATIONS 173 TO 131.

R-5 - (7) HELIBAGS COVERING STATIONS 173 TO 131.	      R-13 - (7) HELIBAGS COVERING STATIONS 173 TO 131.

R-7 - (7) HELIBAGS COVERING STATIONS 173 TO 131.	      R-15 - (7) HELIBAGS COVERING STATIONS 173 TO 131.

R-9 - (7) HELIBAGS COVERING STATIONS 173 TO 131.

SOURCE LINE SHOT ON TODAY:

S-18 - S.P.'s 109 TO 127.

*** OMIT FILE 328; MISFIRE.

*** WIND PICKED UP IN THE AFTERNOON. IT WAS TOO SEVERE TO CONTINUE SHOOTING.

MOVED RECORDER AND TRANSVERSE LINE TO THE NEXT SET UP ON SOURCE 8.

PROFILES FOR THE DAY: 19         				   		  PROFILES FOR THE PROSPECT: 319

TOTAL TRAVEL TIME: 2.0 HRS       FIELD TIME: 10.5  HRS         DOWN TIME: 0.0  HRS        TOTAL CHARGEABLE HRS: 12.5