*** WESTERN GEOPHYSICAL ***

CLIENT:  NORANDA INC.                                    DATE:  SEPTEMBER 18, 1998

AREA:  HALFMILE LAKE 3-D                            JOB NO.:  98-005

RECEIVER LINES:  R-1, R-3, ..., R-15                            PARTY:  779

SOURCE LINES:  S-2,  S-4, S-6 ..., S-32               PARTY MANAGER:  A. ODNOKON

RECEIVER LINE INTERVAL:  406 m.                                      OBSERVER:  R. DAVIS

SOURCE LINE INTERVAL:  406 m.                                      OBSERVER:  G. MITSCHKE

TOTAL RECEIVER LINE KMS.:  48.00					       SOURCE:  DYNAMITE

TOTAL SOURCE LINE KMS.:  44.80			         	   SQUARE KMS:  16.80

SPREAD PARAMETERS                                          INSTRUMENTS

NO.OF TRACES:  1604                                            MAKE:  SERCEL

GROUP INTERVAL:  20 m.                             MODEL & SERIAL NO.:  SN388 P.A.M.'s s/n 17 & 11

SOURCE INTERVAL:  60 m.                                  LOWCUT FILTER:  OUT

NUMBER OF RECEIVER LINES:  8                                       ALIAS FILTER:  0.8 Nyq.,Minimum Phase

NUMBER OF SOURCE LINES:  16                                       PREAMP GAIN:  24 dB.

RECEIVER SPREAD ARRAY:  8 LINES x 201 CHANNELS                 RECORD LENGTH:  3 Sec.

RECEIVER POINT 1ST NO.:  R101                                     SAMPLE RATE:  2 ms.

SOURCE POINT 1ST NO.:  S101                                        POLARITY:  SEG

RECEIVER                                                TAPE

GEOPHONE TYPE:  SENSOR SM-4                                   FORMAT:  SEG-D (8058)

GEOPHONE FREQUENCY:  14 HERTZ                                   TAPE TYPE:  MEMOREX

DAMPING:  60 %                                    TAPE DENSITY:  38,000 BPI

NO. OF GEO'S PER SET:  6 OVER 1.0 m.                        TAPE NO.'S USED:  480842 TO 480900

TYPE OF BASE:  3" SPIKE & 6" CARROT SPIKE

GEOPHONE SEPARATION:  0.2 m. PERPENDICULAR                         SOURCE SYNCHRONIZER

MAKE:  PELTON SHOT PRO

MODEL:  MASTER/SLAVE

AUXILIARIES                                               SOURCE

TAPE            CAMERA                                PATTERN:  SINGLE HOLES

AUX #1:    CLOCK T.B.	      CLOCK T.B.                           HOLE DEPTH:  9 m.

AUX #1:    CONF. T.B.        CONF. T.B.                          CHARGE SIZE:  0.5 Kg.

AUX #1:   UPHOLE TIME       UPHOLE TIME                    TOTAL SHOT POINTS:  702

_________________________RECEIVER ARRAY_______________________       _________________SOURCE ARRAY________________

|              STN                            STN              |     |                                             |

|          x x x x x x                    x x x x x x          |     |     S.P.            S.P.           S.P      |

|                                      -->I I<-- 0.2 m         |	   |      x               x              x       |

|          I<-1.0m-->I                                         |	   |                                             |

|               I<-------------20 m.---------->I               |	   |      I<----60 m.---->I<----60 m.--->I       |

|______________________________________________________________|	   |_____________________________________________|

COMMENTS: DATA QUALITY WAS FAIR TO GOOD ON MOST LINES, WITH NOTICEABLY MORE NOISE APPARENT ON RECORDS AS RECEIVER R-15, R-13 AND R-11

HAD HEAVY ROCK AREAS IN THE MIDDLE OF THE LINES. WEAKER ENERGY WAS ALSO NOTICEABLE IN SOME AREAS. NOISE ON RECORDS WAS CAUSED

MAINLY BY NUMEROUS LOW SOFT AREAS AND HIGH ROCKY AREAS AS WELL AS INTERMITTENT TRAFFIC THROUGH THE MIDDLE OF THE PROSPECT.

WEATHER WAS VARIABLE DURING RECORDING; CLEAR-OVERCAST WITH LIGHT TO MODERATE WINDS AND RAIN THAT SHUT DOWN RECORDING.

TEMPERATURES 9 C TO 20 C.

*** S-20 SHOT POINT 130; CAP LEADS PULLED DUE TO HOLE DRILLED TOO CLOSE TO BORE HOLE.

*** R-1; STATIONS 127 TO 130 WERE DEAD DUE TO BEAVER POND. B.O.L. WAS STATION 127 DUE TO LARGE LAKE AT THE SOUTH END.

*** INTERMITTENT STATIC BURSTS EVIDENT ON VARIOUS SHOT POINTS.