Fig. 15 shows an interpretation of H2 component data from receiver level 10. In general, the location of stratigraphic contacts are in good agreement with the known subsurface geology at the 1600 mine level (Fig. 2). The North Rhyolite contact is clearly visible as an arcuate reflection that crosses the image region. As expected from the well log data (Fig. 7), several prominent reflective packages are visible in the footwall. The anomalous dipping reflection noted earlier is truncated by the North Rhyolite reflection near the centre of the figure (white arrow). Unfortunately, two gaps in the line of shots (Fig. 2) that occur where the road crosses a drilling pad produced narrow regions where there is no data in the mapped image. These thin slices interrupt the continuity of the dipping reflection. Nevertheless, the attitude and location of this event indicates that it is most likely associated with a wedge-shaped segment of the North Rhyolite that has been delineated by drilling. The existence of anomalously high-amplitude seismic reflections, as indicated in Fig. 8, implies that this region is characterized by strong impedance contrasts. These inferred strong impedance contrasts, coupled with a generally favourable structural/stratigraphic setting, suggest that this segment of the North Rhyolite may contain massive sulphides.

Other cross-cutting relationships visible in the seismic data help to constrain the interpretation. A series of reflections with a common WNW strike appear to terminate along a NNW surface, as indicated by the arrow near ... Comparison with geologic data from the 1600 mine level suggests that this truncating surface represents the base of a mafic volcanoclastic unit, and that the WNW trending reflections are probably gabbroic sills. The impedance contrast between the North Rhyolite and the overlying gabbro sills leads to a conspicuous reflection that can be readily identified in the seismic image. The boundary between the Rhyolite and the ultramafic rocks (talc-carbonates) appears to be less reflective. This difference in reflectivity provides a possible basis for distinguishing between the Rhyolite-ultramafic contact and the Rhyolite-gabbro contact, and thus map the distribution of the ultramafic unit.

Data from the H1 component (Fig. 14), more sensitive to structures south of the receiver array, indicate an abrupt change in the strike of the North Rhyolite and associated footwall reflections, from WNW to WSW. This abrupt bend occurs along a well defined N-S axis, and is interpreted as small-scale folding or faulting in the vicinity of the borehole.

Although the overall agreement in the shape and orientation of structures between the seismic image and the 1600 level geology is excellent, the registration is not exact and some discrepancies exist in the location of subsurface reflectors. For example, if the proposed correlation of the anomalous dipping reflection with the wedge-shaped protusion of the North Rhyolite is correct, then the seismic reflections have been misplaced by approximately 50-100 m to the south of their true location. This discrepency arises from innacuracies in the transformation procedure used to map the seismic data into mine coordinates. For example, the use of a high velocity (6950 m/s) characteristic of mafic rocks near the receiver array leads to a slight stretching out of the much slower (~5500 m/s) Rhyolite zone. To improve the three-dimensional mapping of seismic reflections, more sophisticated model-driven techniques are required. In principal, an iterative modelling approach could be used to update the velocity field and reflector configuration in order to obtain the best fit to the observed reflection times, using the simple mapping result as a starting model.

Because three-component measurements of particle motion are available (H1, H2 and V), the seismic data furnish other constraints that can be used to test the validity of the model. If contamination by S waves has been completely removed in the basic processing, then we can expect the orientation of the P-wave particle-motion vector to point toward (or away from) the body where the reflection originated. Commonly used seismological techniques for particle-motion studies, such as hodogram analysis (ref), are not well suited to the investigations using large datasets. Here, a simple alternative method for estimating and plotting particle-motion direction is used. Using time picks obtained by scanning through the data volume (Fig. 7), the RMS amplitude for a window that straddles the North Rhyolite contact is computed from the H1, H2 and V traces. These RMS values provide orthogonal components that define an estimated particle-motion vector, which is then normalized and plotted in a stereographic projection. Fig. 16 shows the results of this particle-motion analysis for the 83 shots for receiver level 1. Althought the results are still preliminary, this graph shows that: 1) observed particle-motion vectors change in a systematic fashion over the range of shots; 2) the overall direction of particle motion is consistent with subhorizontal arrivals from the SW; 3) the angular aperture implied by the particle-motion directions is consistent with the transformed images (Fig. 14); and 4) from shot 1 to shot 83, the reflection point swings to the NW along the North Rhyolite contact, consistent with the assumptions underlying the transformation technique. The particle-motion analysis is entirely independent of other approaches to determining the location of reflectors, since it is based on the relative contribution of the signal from the three receiver components rather than arrival time. This vector information is also predictable from modelling, and so could be used to guide a model-based imaging procedure.

Conclusions and Recommendations

This report describes the sucessful implementation of an innovative side-scanning borehole seismic survey, designed to image along the strike of the North Rhyolite, a steeply dipping stratigraphic contact near the northern flank of the Kidd Creek volcanic complex. The results presented here build on the results of the previous two phases, and confirm several of the previous conclusions:

1) Vertical-seismic profiling (VSP) technology can be tailored to provide detailed three-dimensional images of steeply dipping panels of contrasting lithologies;

2) The side-scanning data provide new direct evidence that massive sulphides generate prominent seismic reflections.

This work has led to the development of new, state-of-the-art technology for borehole seismic acquisition and processing. The processed seismic data have yielded pie-shaped images representing subhorizontal sections at about the 1600 mine level, with data coverage that extends over a maximum along strike distance of about 800 m. The North Rhyolite is well defined by the reflection images, with signal penetration into its footwall of at least 500 m. The geometry and configuration of reflections is in excellent agreement with the known geology and the 1600 mine level. Slight discrepencies between the locations of reflectors in the mapped images and the 1600 level geology can be attributed to approximations used in the mapping algorithm. Other independent methods, such as iterative forward modeling and particle-motion analysis, could be developed to refine the location of seismic reflectors in three dimensions, potentially with sufficient accuracy to deliver precise drilling targets from seismic anomalies. An anomalous, high-amplitude seismic reflection observed on the H2 component between receiver levels 7 and 13 is interpreted to originate from massive sulphides developed within a wedge-shaped protrusion of the North Rhyolite.

Removal of direct P-wave and S-wave arrivals is a critical element of the basic processing stream. Experience from this IPP project indicates that a minimum of 13 receiver levels are required for optimal functioning of the multichannel filters used to accomplish this task. We therefore recommend that a minimum of 13 receiver levels should be used for side-scanning borehole surveys. Additional levels could be added to provide extended vertical coverage of the target horizon.

Where VSP acquisition levels intersect a known stratigraphic marker (e.g. the North Rhyolite), it is possible to identify, without ambiguity, the corresponding reflection in the data. This presents a major advantage over surface-seismic methods for interpreting the data, since the interpretation can then be "hung" from this marker. However, in order to scan a large area, the side-scanning receiver array should be separated from the target horizon by a significant distance (several hundred metres). We therefore recommend that side-scanning borehole surveys be accompanied by a deep conventional VSP survey that passes through the side-scanning receiver array and itersects the stratigraphic marker, to assure accurate correlation of seismic reflections with geologic marker horizons.

Although significant potential for this method has been demonstrated in the current IPP program, the side-scanning borehole seismic technology lags far behind surface-seismic methods in its develpment and is most properly regarded as an area for continued research and development. Small explosive charges in shallow holes (side-scanning method), repeat shots in water-filled pits (VSP) and procedures used in the basic data processing stream have proven robust and effective, but methods to reduce the cost of acquistion would make the technique more attractive for routine exploration. Some possible ways to achieve this would be to develop multilevel borehole seismic sensors, or downhole sources that would permit the acquisition of data in a "reverse-VSP" configuration. The 3-D imaging step, and other advanced data processing techniques, can be refined to provide more accurate position of reflectors. Because of the possible site-specific nature of this procedure, presurvey seismic modelling to determine optimum shooting geometry is strongly recommended. Additional tests of the side-scanning seismic method will help to establish its sensitivity and robustness for massive sulphide deposits in different settings.

Appendix A

Raw Data

The following plots show the full side-scanning dataset, after being merged with the single receiver level recorded during phase II and sorted into common shot gathers.

Appendix B

Data volume after basic processing

The following plots show the full data volume for the side-scanning survey, after application of basic processing to 1) remove the direct P-wave and S-wave arrivals and 2)

Appendix C

Transformed Data

The following plots show the full side-scanning data volume, after transformation into mine co-ordinates. The input to this step is the data shown in Appendix B. A mathematical description of the transformation operator is given in Appendix D.

Appendix D

Formulas for transforming side-scanning seismic data to mine coordinates

The transformation operator used to map the side-scanning data from acquisition coordinates (shot-receiver-time) to mine coordinates (x,y,z) assumes that reflectors in the subsurface dip at a uniform angle, θ. It is also assumed that, for a given shot-receiver pair, all raypaths are coplanar. In map section, this means that a line passing through the shot and receiver will intersect the curved reflection interface at a right angle. Finally, straight raypaths (i.e. constant velocity, v) are assumed.

These assumptions lead to a simple raypath geometry (Fig. D-1) that is equivalent to the 2-D geometry for a dipping reflector recorded at the surface. The angle between the dipping reflector and a line passing through the shot and receiver is denoted by ψ. The traveltime for reflections to arrive at the receiver is given by

where va is the apparent velocity, given by v/cos ϛ, and h is the half offset. The zero-offset time is given by t0 = d0/v.

To implement the mapping procedure, we begin by expressing d0 in terms of known quantities:

This determines the location of point B with respect to the midpoint A between the shot and receiver. The reflection point (C) is offset from B by the amount (Levin, 1971)

A given data value (for which A, h and t are known) can then be mapped into image (mine) coordinates at the point C.

For the results shown in Appendix C, a dip θ = 70o was assumed and the measured velocity of the first arrivals (6950.0 m/s) was used for the transformation. The mapping operation was restricted to causal arrivals (those that arrive after the direct P wave). It was also necessary to account for near-surface delays by subtracting a constant value of 27 msec from the measured traveltimes. This is the intercept time in the plot of first-arrival time picks vs. distance (Fig. 5).

Appendix E

Shallow VSP Survey in Borehole BH-