Reflection seismic data from the Sturgeon Lake Mining Camp, Northern Ontario

Brian Roberts and Erick Adam

Continental Geoscience Division, Ottawa

Project number: 970012

Introduction

There are an increasing number of studies which demonstrate the benefits of using the seismic technique in existing mining camps, and in particular show that it is a cost effective way to map favourable mine horizons (e.g. Milkereit, et al., 1996, Perron, et al., 1997) and, potentially, is capable of directly detecting massive sulphides (e.g. Adam et al., 1997). It has been shown that the success of seismic studies in a crystalline environment is dependent on prior knowledge of the structures (i.e. dip, strike, thickness) and the physical properties (i.e. density and velocity) of the host rocks and mineralization.  This information is obtained by geophysical logging in existing boreholes, performing lab measurements on rock samples, and examining the geological database.

In November 1997, Noranda Mining and Exploration Inc. took the opportunity to utilize a seismic crew that had been mobilized from Calgary for the acquisition of the LITHOPROBE Western Superior seismic transect to acquire a 7.8 km seismic line near several existing mining camps in the Sturgeon Lake greenstone belt, Northern Ontario (Figure 1).  The area is well mapped and Noranda has a number of boreholes in the vicinity of the seismic line to aid in the interpretation of the data.

Processing of the seismic data was undertaken at the Seismology and Electromagnetism Section of the Geological Survey of Canada using the ProMAX seismic data processing software.  Detailed geological information was not available and as a result this report outlines the processing performed and displays the final sections along with a description of the main features in the data rather than providing a complete interpretation of the data.

Geological Setting

The Sturgeon Lake greenstone belt lies in the Superior Province at the boundary between the central and western Wabigoon Subprovince where Mesoarchean crust (central Wabigoon) appears to lie adjacent to younger supracrustal belts (western Wabigoon) (Thurston, et al., 1991).  Greenstone belts (ca. 2.73 Ga) are widespread in the Wabigoon Subprovince and host numerous base-metal mineralization occurrences. The area of interest for this study has been mapped in detail and the zone which contains the known mineral occurrences has been interpreted as an Archean submarine caldera complex (Morton et al, 1991).  The caldera volcanics overly a pre-caldera volcanic cycle to the south and are overlain by younger mafic volcanic rocks to the north (Figure 1).  All the metavolcanic units in the Sturgeon Lake mine camp area are dated in the 2.70 - 2.735 Ga range.

Seismic Data Acquisition Parameters

Receivers along the entire line were active for all the shots, and 3 seconds of 393 channel data were recorded with a sampling rate of 1 ms for each of the 223 dynamite shots.  The 393 trace spread used a receiver spacing of 20 m and the shot spacing was 40 m.  A visual inspection of the shot gathers indicated that the dynamite shots provided ample source energy with strong first break energy along the entire spread of receivers and obvious reflection energy is seen at 1.5 and 2.5 seconds.

Data Processing

The processing of seismic data can be divided into those steps which are performed prior to stacking the seismic traces (referred to as prestack) and those done after stack or poststack.

Prestack

The prestack processing of the shot gathers focusses mainly on the elimination of noise, both source generated and ambient background, the application of static corrections, and the determination of the correct velocity function for stacking the data (including the application of dip moveout correction).  Noise elimination is critical because in the crystalline environment we are dealing with low reflection coefficients and thus, a low signal-to-noise ratio.  Below we have listed the highlights of the prestack processing with applicable comments.

1) Geometry definition

The seismic line was not straight (Figure 1), however, a smooth binning line could be fitted through the source-receiver midpoints such that most of the common depth point (CDP) bins contain a full range of offsets.  The CDP fold is as high as 224 in the middle of the line, smoothly tapering to a low fold at the ends of the line.  Elevations vary by only 10 m along the length of the line.

2) True amplitude recovery

To correct for amplitude loss due to spherical divergence the data were scaled using a time variant factor [1/(time*vel**2)] with a constant velocity of 6000 m/s .  As well, a full trace (3 seconds) automatic gain function was applied to all traces in order to correct for variations in source and receiver coupling.

3) Deconvolution

Deconvolution improves the temporal resolution of seismic data by compressing the seismic wavelet.  After testing a minimum phase spiking deconvolution was performed using an operator length of 120 ms.

4) Bandpass filter

The low frequency end of the spectrum primarily contains the dispersive ground roll energy and  air blast (see Figure 2A) along with ambient noise related to workings at the mine site, small streams, etc.  To remove this energy the low cut of the bandpass filter was set at 32 Hz.  The  shot gathers also contain abundant high frequency energy, beyond  200 Hz, but it was observed that the reflective energy in the shot gathers was below 150 Hz.  The bandpass filter was thus set to be quite wide, between 32-150 Hz, at this early stage of the processing.

5) Elevation statics.

The data are corrected to a flat datum of 430 m using a replacement velocity of 5900 m/s.

6) 2-D spatial filtering

After deconvolution and bandpass filtering, there still remains a relatively strong, but discrete, refracted shear wave arrival (head wave) (Figure 2A).  In order to remove as much of this as possible we applied a median filter which is focussed along a specific velocity (+/- 3450 m/s in this case).  Strong energy travelling at that velocity should be much reduced.  This process was about 80% effective, but it was anticipated that the dip moveout (DMO) correction would attenuate the remaining coherent shear wave energy.

7) Mute first break energy

Trace muting is based on the first break picks.  The data is muted from 30 ms below the first break pick to the top (time 0).

Figure 2B displays the top 1 s of a processed shot gather at this point of the processing sequence.  The coherent noise has been effectively removed and the reflectivity, although weak, can now be identified.

8) Refraction statics application

The Canadian shield typically has ground conditions that vary from bedrock at surface to  variably thick overburden and an unpredictable height for the water table.  Seismic waves travel slowly through overburden, especially if it is dry, and are consistently fast in bedrock.  Therefore, along the length of a receiver spread a variation in the travel times to the receivers is normally observed which is related to the thickness of the weathering layer at each location.  This static correction is vital in obtaining a good seismic image in any mining camp scale seismic survey.  In this case, a one layer refraction statics model was defined using the uphole times from the shots and the ProMAX refraction statics package.  Three methods were used for the computations (Diminishing Residual Matrices (DRM), Generalized Residual Method (GRM), and delay times) and all came up with comparable results.  The depth of overburden model and the source and receiver statics used to correct for the weathering layer are shown in Figure 3.

9) Residual statics

The lateral continuity of reflection energy can be improved prior to the stacking process by calculating source and receiver statics which yeild a maximum CDP stack power over a given window on each CDP gather.  This process can be effective in low signal-to-noise data.  Surface consistent residual statics were calculated using a 600 ms window centred on the strong package of reflectivity at approximately 1 second.

10) Dip moveout correction (DMO)

In complex geological environments reflections with various dips are commonly observed and this is true at Sturgeon Lake.  The dipping events are typically related to steeply dipping contacts/faults, or diffractions from structural discontinuities and high impedance scatterers.  In applying the normal moveout correction there is no velocity function that will simultaneously optimize stacking of both dipping and sub-horizontal events.  The dipping events stack at anomalously high velocities.  To circumvent this problem, DMO is used to effectively create a zero-offset section, in which dipping events will stack at realistic velocities.  Prior to DMO an average background velocity (6000 m/s) is used to correct the data, then a full velocity analysis is performed following DMO.

11) Velocity Analysis

The velocity analysis was performed using a combined semblance/constant velocity stack display every 50 CDP's (500 m).  The velocities varied somewhat along the line, but on average the velocity function varies from 5600 m/s at the surface to 6350 m/s at depth (see Table 1).

Time (ms)	RMS Velocity (m/s)

50.0		5600

300.0		5900

500.0		6000

1000.0	6100

2400.0	6350

Table 1.  Typical velocity function used for NMO correction of Sturgeon Lake line 1.

11) Normal moveout (NMO) correction and CDP stack

Following the NMO correction the data are stacked using a straight mean stack with a maximum fold of 224.

Poststack

12) Bandpass filter

Up to this point we have maintained a wide frequency spectrum in the data.  This is to ensure that every chance is given to enhance the high frequency data, especially in the top .5 s.  In analyzing the data poststack it is observed that most of the coherent energy lies in the 30-80 Hz range, with some higher frequency signal (up to 120 Hz) in the upper part of the section.  Because of this, a time variant bandpass filter was applied at this point (0-.3 s, 30-120 Hz; .3-.45 s, 30-100 Hz; .45-3.0 s, 30-80 Hz).

13) FK migration

Stolt F-K migration with a constant velocity of 6000 m/s (average background velocity).

14) Automatic Gain Control (AGC)

A scale factor is calculated using a 1 s window and applied to the sample at the centre of the window.

Observations from the stacked and migrated sections

The data acquired at Sturgeon Lake are of high quality and the data processing was fairly routine with no major problems encountered. The geological information required to do a meaningful interpretation was not available, however useful observations can be made about the major features displayed on the seismic section.

Below 1 s on the DMO section (Figure 4) we see a series of dipping interfaces (apparent dip of 20o to 30o NE) and a strong sub-horizontal reflection package at 2.5 s.  The dipping events are of varying strength, but continuous from stn 463 to the northeast end of the section.  The exception to this is the lowest layer of dipping reflections which is strong at the southwest end of the line, but becomes incoherent at approximately the halfway point (around stn 250).  Above 1 s the picture is more complex with strong but patchy reflectivity in the southwest end of the line and a more transparent section to the northeast (at least above .4 s).  At least 2 diffractions that can be identified; one with an apex at station 220 and .5 s (A on Figure 4), and one at station 345 and .8 s (B on figure 4).  The first diffraction (A) also appears to have a series of weaker diffractions below it, but they are not so continuous.  Also of note in the top part of the section are some weak, steep northeasterly dipping reflections (approx. 40o dip) between stations 240 and 290 and 150 - 400 ms (C on figure 4).  The sub-horizontal reflections which make up the strong reflectivity in the west part of the line appear to truncate rather abruptly as they extend east.  A hypothetical line (see dotted line on figure 4) joining these truncations (a fault?) would extend to surface at about station 420, near the contact between the older mafic volcanic group and the intrusive volcanics to the south (Figure 1).

The migrated section (Figure 5) is noisier and contains more artifacts than the DMO section, however, it does clarify the picture somewhat in the upper section.  Below 3 km the events are as described above except that the dipping events have moved up dip to the southwest and appear to flatten towards the northeast end of the line.  Above 3 km we see that the 2 diffractions mentioned above have collapsed to fairly discrete zones of moderate to high amplitude reflections.  The dipping reflections between stations 240 and 290 (referred to as C above) have steepened up and now we can project them from the surface at approximately station 315 down to a package of sub-horizontal reflections at 1.5-1.8 km (stations 210-220).  Again, the sub-horizontal events to the west end of the line appear to truncate against some east-dipping plane.

Equivalent Offset Migration

Equivalent offset prestack migration (EOM) has been applied to Sturgeon Lake line 1 using an ITA/Insight module developed at the GSC.  Theory and various application of equivalent offset migration are described by Bancroft et al. (1998).  For a constant velocity the typical NMO, DMO, CMP stack process is kinematically equivalent to prestack migration (Hale, 1984).  However, EOM provides an efficient method for acquiring detailed velocity information and comparisons with poststack migrations have shown that equvalent offset migrations have better coherency and hence, are more interpretable (Bancroft, 1998).  In the application of this method common scatter point (CSP) gathers are assembled at the CDP locations along the processing line.

A CSP bin width of 20 m (twice the CDP spacing) was selected based on receiver spacing and trial-and-error testing with other data, generating 694 CSP gathers.  Each CSP gather consisted of 500 traces with equivalent offsets between 0 and 10 000 m (Figure 6).  Note that different bin sizes were not tested.  The migration velocity used to assemble the CSP gathers was 6.0 km/s and the maximum migration time is 3 s.  The 3 s maximum migration time and the large offsets (10 km) were selected to avoid excessive smearing at depth.

The input traces for the CSP migration have been pre-processed using the pre-stack processing flow described above up to and including the DMO correction step.  After the CSP gathers were assembled, NMO corrections and stacking were performed using ITA/Insight software.  A single stacking velocity function was used: 0.0 s - 5500 m/s, 0.2 s - 5900 m/s, 0.5 s - 6000 m/s.  A NMO stretch mute factor of 50 % was selected as a compromise between frequency content at shallow depth and imaging of shallow reflections.  Stack sections using 0 and 30 % stretch mutes have been produced and showed strong sub-horizontal events at the near surface which were probably caused by remnants of unmuted first break energy.

The EOM prestack migrated section of the Sturgeon Lake line (Figure 7) overall shows better  continuity of events between 200 and 3000 ms than the FK migrated section (Figure 5). However, the migrated DMO section shows better images in the top 600 m and we attribute this difference to the wide aperture of the equivalent offset migration.  Data points having an aperture close to 90o often include remnants of first break energy, creating high amplitudes on the CSP gathers.  These were attenuated by applying spectral equalisation with a 500 ms sliding window.  While this approach has greatly reduced the noise some is still visible in the top 600 m and the data can no longer be considered "true amplitude".

Discussion and Conclusion

We have observed strong reflectivity at Sturgeon Lake and some potentially interesting structures, both at depth and in the top .5 seconds.  It is possible to compare this data to the LITHOPROBE regional seismic data that also crosses part of the Sturgeon Lake greenstone belt.  The southern end of regional line 1A extends into the north end of the belt for roughly 30 km ending approximately 40 km northeast of Sturgeon Lake line 1.  The most prominent reflector in the upper crustal part of line 1A is a strong south dipping package that lies at 2.5-4.0 s (White et al., 1999).  This event may correlate with the strong sub-horizontal reflection we see at 2.5 s on the high resolution line and indicate that this is a regional reflection underlying the greenstone belt.

The final stacks (DMO structure stack and migrated stack) could possibly be improved by more effort in at least 2 areas.

1) The refraction statics solution appears to be at least adequate, but it may be that a better solution could be arrived at through the use of a more robust refraction statics program.  It is possible that this would lead to an improved image in the upper section.

2) The velocity analysis could be redone using the common scatter point gathers.  The large equivalent offsets of the CSP gathers and better signal-to-noise ratio than standard common midpoint (CMP) gathers translate into a more accurate velocity analysis.  This would result in minor tuning of the velocities leading to a more accurate picture of the true velocities, and perhaps a slightly better image.

The key processing steps for these data were bandpass and 2-D spatial filtering (coherent noise removal), refraction and residual statics (improved signal continuity), and DMO (to enable the definition and application of the correct velocity function). In this case, the application of a prestack migration was worth the computational overhead and we would recommend the use of Equivalent Offset prestack migration for all short and moderate length, high resolution seismic lines in mining camps.

Acknowledgments

The seismic data was collected by J.R.S. Exploration Co. Ltd. of Calgary.  We are grateful to Noranda Mining and Exploration Ltd. for allowing the publication of these data.  Azad Rafeek provided invaluable drafting assistance and Ian Kay supplied the inset for Figure 1.

References

Adam, E., Arnold, G., Beaudry, C., Matthews, L., Milkerriet, B., Perron, G. and Pineault, R.

1997:

Seismic exploration for VMS deposits, Matagami, Quebec, in Proceedings of Exploration 97: Fourth Decennial Conference on Mineral Exploration, edited by A.G. Gubins, 433-438.

Bancroft, John

1998:

The equivalent offset method of prestack time migration, Geophysics, 63, No. 6, p. 2042-2053.

Hale, D.

1994:

Dip Move-out by Fourier transform, Geophysics, 49, p. 741-757.

Milkereit, B., Eaton, D., Wu, J., Perron, G., Salisbury, M., Berrer, E. and Morrison, G.

1996:

Seismic imaging of massive sulphide deposits, part 2: reflection seismic profiling, Economic Geology, 91, 829-834.

Morton, R.L., Walker, J.S., Hudak, G.J., and Franklin, J.M.

1991:

The early development of an Archean submarine caldera complex with emphasis on the Mattabi ash-flow tuff and it's relationship to the Mattabi massive sulphide deposit; Economic Geology, v. 86, p. 1002-1011.

Perron, G., Milkereit, B., Reed, L., Salisbury, M., Adam, E. and Wu, J.

1997:

Integrated seismic reflection and borehole geophysical studies at Les Mines Selbaie, Quebec: CIM Bulletin, September, 75-82.

Sanborn-Barrie, M., Skulski, T., and Whalen, J.B.

1998:

Tectonostratigraphy of central Sturgeon Lake, Ontario: deposition and deformation of submarine tholeiites and emergent calc-alkaline volcano-sedimentary sequences; in Current Research 1998-C; Geological Survey of Canada. p.  115-136.

Thurston, P.C., Osmani, L.A., and Stone, D.

1991:

Northwestern Superior Province: review and terrane analysis; in Geology of Ontario, (ed) P.C. Thurston, H.R. Williams, R.H. Sutcliffe, and G.M. Stott; Ontario Geological Survey, Special Volume 4, Part 1, p. 81-144.

White, D., Helmstaedt, H., Harrap, R., Thurston, P., van der Velden, A., Hall, K., Davis, D.

1999:

Accretionary tectonics in the late Archean? First results from the LITHOPROBE Western Superior transect, Western Superior transect workshop, Ottawa.

Figure Captions:

Figure1:

Map showing the location of Sturgeon Lake seismic line 1 and the regional setting of the Sturgeon Lake mine camp area within the Archean Superior province (inset).  Geology is modified from Sanborn-Barrie et al. (1998).

Figure 2:

A display of the top 1 s of a raw (A) and processed (B) shot gather from station 255 on line 1  The weak reflectivity is apparent on B after the removal of all the coherent noise.

Figure 3:

Shot and receiver refraction statics (top) and depth of overburden (bottom).

Figure 4:

Sturgeon Lake line 1 dip moveout (DMO) stack (1:1 scale). A and B indicate diffractions, C points to steeply dipping, near surface reflectivity, and the dotted line joins the apparent truncations of the horizontal reflectivity seen in the SW part of the section.  The arrows indicate reflections referred to in the text.

Figure 5:

Sturgeon Lake line 1 migrated stack (FK migration at 6.0 km/s).  See figure 4 for description of  features.

Figure 6:

Common scatter point (CSP) gather at station 330.  Offsets between 0 and 8 km are displayed.  A 30-130 Hz spectral equalization with a 1.5 s window has been applied  Arrows indicate reflections observed at various depths.

Figure 7:

Equivalent offset migration stack of Sturgeon Lake line 1.